# Plan

Project Title:

Hardware Accelerated YOLO on PYNQ for Smart Traffic Systems

Objective:

Improve the size of YOLO-based object detection, with negligible losing accuracy (traffic)

| Week | Tasks |
| --- | --- |
| 1 | 1. Prepare the knowledge needed for the project 2. Plan for the tasks |
| 2 | 1. Toolchain setup (e.g., TF, TF Lite, PyTorch) 2. YOLO baseline model + dataset selection based on literature |
| 3 | Layer by layer analysis of the baseline YOLO based on the literature (and our analysis) |
| 4 | Model optimization analysis  Run a YOLO model on CPU/ GPU  Layer-based error injection and profiling |
| 5 | Model optimization methods exploration  Quantization  Pruning |
| 6 | Profiling + bottleneck analysis  layer-wise latency  memory  FPS  CPU usage |
| 7 | Run YOLO inference on an embedded platform (e.g., ARM) |
| 8 | Hardware / software partitioning Which layers should be run on FPGA |
| 9 | 1. Resource utilization 2. Latency & throughput measurement |
| 10 | Comparison vs software baseline (accuracy vs speed trade-off) |
| 11 | 1. Power & efficiency analysis 2. Final evaluation |
| 12 | 1. Prepare figures, tables on what I worked 2. Draft and work on research paper |

# Week 1: 25-29 May 2026

Since I am completely new to this topic, I learned a few things about it before working on the project. Here is what I learned:

1. Image processing example (in traffic system)

| Image Classification - There is a car - Confidence: 98% DNN models Residual Network - accurate Efficient Net - balanced accuracy and speed More examples Input A traffic sign Output Is a stop / speed limit / yield sign Input A road of car Output It is low / medium / high traffic Main use Recognizing objects, e.g.: - traffic sign - road crowd level - road weather | Image Detection - There is a car - Position: x=256, y=128 - Size: width=500, height=250 - Confidence=98% DNN models YOLO - one stage detector - very fast Faster R-CNN - accurate More examples Input A Street photo Output the position, size of: - cars - human - traffic lights - etc. Main use Detect objects with their position and size, e.g.: - cars - human | Image Segmentation - There is a car - including pixels: […] DNN models U-Net - good at area separation Mask R-CNN - good at shaping objects More examples Input A street photo Output Separate the Area of - Road for cars - Road for humans - Roadblock - Sky - etc. Main use Splitting different areas, e.g.: - cars can drive area - humans cannot walk area |
| --- | --- | --- |

In this project, we will focus on image detection, with the DNN model YOLO.

2. YOLO (you only look once)

| An image will be separated into tiles, each tile has: [Pc Bx By Bw Bh C1 C2] to represent a box | - Pc is the confidence that the object exists - Bx, By are the x, y coordinate of the object - Bw, Bh are the width and height of the object - C1, C2 are the types of the object |
| --- | --- |
| For overlapping boxes, divide the intersection area by the union area, and remove the one with a lower Pc if the result is high. | For more objects in the same grid, the vector can simply be expanded: [Pc … Pc …] |

2.1 CNN

CNNs include many layers, and after going through those layers, the image becomes informative. One of the goals of this project is to reduce the number of layers without losing accuracy.

2.1.1 Convolution layer

An image can be represented as a matrix of pixel values. A filter matrix is moved across the image. At each position, the filter is multiplied with the corresponding image window element by element.

After the filter is applied to every position in the image, the output is a feature map. Larger values in the feature map mean stronger matching pattern in that part of the image.

Different filters can detect different types of features, such as vertical edges, corners, circle-like shapes, or textures.

In traditional image processing, these filters can be manually designed. In CNNs, the filters are usually learned automatically during training.

With this knowledge, I tried a mini program:

| Input | Process | Output |
| --- | --- | --- |

2.1.2 1x1 Convolution layer

Used to reduce the number of feature maps.

2.1.3 Activation layer (Leaky ReLU)

Scan values in a feature map

- if positive value: keep it

- If negative value: multiply it by 0.1 (or a number between 0 and 1)

2.1.4 Max pooling layer

Simplify by taking the maximum value in a window, e.g.:

[1, 2, 3, 4] => [4]

2.1.5 Fully Connected Layer

The fully connected layer combines all features and converts them into the final prediction output. The format can be [Pc, Bx, By, Bw, Bh, C1, C2] as mentioned before. It can be further flattened to a long vector. For example, YOLOv1 is 7×7×30.

3. Process of YOLOv1 CNN

Later, I may compare the latest YOLO model with YOLOv1 to see what it changed to increase speed while maintaining accuracy.