# Week 10: 27-31 July 2026

Table of Content

| Content | Page |
| --- | --- |
| 1. Competitor: Snow Aware (CF YOLO) | 2 – 8 |
| 2. Competitor: Traffic YOLO Pruning (BDD100K) | 9 – 18 |
| 3. Negative AD Reasoning (with paper support) | 19 – 20 |
| 4. Negative AD Handling (with paper support) | 21 – 23 |
| 5. PYNQ-Z2 PS-Side Baseline Evaluation | 24 – 27 |
| 6. Pruning: Layer ablation using parameter-free replacement | 28 – 33 |
| 7. Pruning: Global L1 channel-magnitude (pending) | 34 – |
| 8. Our Formular | 35 |
| 9. Summary | 36 – 37 |
| 10. My overall notes | 38 |
| References | 39 |

→

1. Competitor: Snow Aware (CF YOLO) [1]

1.1 Overview

CF-YOLO was proposed by Ding et al. in the peer-reviewed journal IEEE Transactions on Intelligent Transportation Systems.

The study addresses object-detection under snowy conditions. It introduces a real-world snow object-detection dataset named RSOD and proposes a lightweight detector called CF-YOLO, which is constructed by replacing the original YOLOv5s neck with Cross Fusion blocks.

1.2 Main Contributions of CF-YOLO

The paper makes three main contributions.

1.2.1 RSOD Dataset

The authors establish RSOD: Real-world Snowy Object Detection Dataset, containing 2,100 real snow images. Most images show townscape and traffic scenes, and annotations are supplied in both COCO and YOLO formats.

1.2.2 Snow Coverage Rate

The paper defines the Snow Coverage Rate, or SCR:

SCR=AsnowAbbox

where:

- Asnow is the estimated snow-covered area inside an object;

- Abbox is the area of the object bounding box.

SCR is combined with human observation to divide RSOD into four difficulty levels.

1.2.3 Cross Fusion Block

CF-YOLO replaces the FPN and PANet-based neck of YOLOv5s with Cross Fusion blocks.

Instead of passing features only through sequential top-down and bottom-up paths, the CF block directly combines features from several backbone stages.

The intention is to recover low-level information such as edges, outlines and texture that may be weakened or hidden by snow.

CF-YOLO uses two stacked CF blocks, while the convolution kernel size K can be set to 1 or 3.

K=1 focuses on cross-level feature fusion using 1×1 kernels, while K=3 uses 3×3 kernels to capture additional information.

1.3 RSOD Dataset Usage

| Item | RSOD configuration |
| --- | --- |
| Total images | 2,100 |
| Total annotated objects | 7,862 |
| Image conditions | Real-world snow |
| Main scene type | Townscape and traffic |
| Annotation formats | COCO and YOLO |
| Training set | 1,701 images |
| Validation set | 189 images |
| Test set | 210 images |
| Training proportion | 81% |
| Validation proportion | 9% |
| Test proportion | 10% |
| Snow difficulty levels | Four |

Table 1.1: Dataset Summary

Table 1.1 shows that RSOD is a relatively small dataset. The authors randomly allocate images to the three subsets while attempting to balance the snow-difficulty distributions.

| Class | Number of annotations |
| --- | --- |
| Person | 3,159 |
| Car | 2,606 |
| Truck | 451 |
| Traffic light | 443 |
| Umbrella | 419 |
| Handbag | 268 |
| Backpack | 147 |
| Bicycle | 126 |
| Bus | 99 |
| Other classes | 144 |
| Total | 7,862 |

Table 1.2: Class Distribution

Table 1.2 shows person and car account for most annotations. Trucks, traffic lights, bicycles and buses are also relevant to traffic, although several classes such as umbrellas and handbags may not be necessary in traffic object detection.

1.4 Snow Difficulty Levels

| Snow difficulty | Images | Dataset share | General description |
| --- | --- | --- | --- |
| Easy | 600 | 28.6% | Objects remain relatively clear |
| Normal | 1,000 | 47.6% | Noticeable snow degradation |
| Difficult | 400 | 19.0% | Substantial snow coverage |
| Particularly difficult | 100 | 4.8% | Severe snow coverage |
| Total | 2,100 | 100% |  |

Table 1.3: RSOD four levels

For Table 1.3, the paper indicates that difficult samples generally have SCR between 0.5 and 0.7, while particularly difficult samples generally have SCR greater than 0.7. Difficulty is mainly assigned through human observation, with SCR used when the level is uncertain.

It demonstrates that treating all snow images as one condition may hide differences between light and severe snow. (this could be the reason we separate anow and non-snow)

1.5 Experimental Configuration

CF-YOLO is implemented in PyTorch. The reported training settings follow YOLOv5s:

| Setting | Value |
| --- | --- |
| Batch size | 32 |
| Optimizer | SGD |
| Momentum | 0.937 |
| Weight decay | 0.0005 |
| Initial learning rate | 0.01 |
| RSOD fine-tuning | 20 epochs |
| MSCOCO training | 300 epochs |
| Speed-test GPU | Tesla V100 |
| Speed-test batch size | 1 |
| Speed-test sample size | 5,000 COCO val2017 images |

Table 1.4: settings

For Table 1.4, the paper does not clearly report the input-image resolution in its experimental section. Therefore, the report should not assume that it used 640 × 640 unless this is verified from the released configuration files.

1.6 Results under Different Snow Difficulty Levels

The first experiment trains all models only on MSCOCO and evaluates them on RSOD. Therefore, this experiment measures how a general detector performs under real snow without RSOD-specific fine-tuning.

The paper reports AP and AP50. To avoid metric ambiguity, the report should retain the authors’ term “AP” rather than automatically renaming it mAP50–95.

| Method | Easy AP / AP50 | Normal AP / AP50 | Difficult AP / AP50 | Particularly difficult AP / AP50 | All levels AP / AP50 |
| --- | --- | --- | --- | --- | --- |
| YOLOv5s | 41.1 / 61.3 | 36.1 / 55.5 | 26.4 / 41.3 | 25.2 / 39.2 | 34.3 / 52.0 |
| YOLOv6-nano | 39.9 / 57.5 | 35.7 / 51.7 | 32.0 / 44.9 | 29.1 / 45.3 | 33.6 / 49.1 |
| YOLOv7-tiny | 41.5 / 61.9 | 38.7 / 57.7 | 27.8 / 43.9 | 27.3 / 52.4 | 35.9 / 53.9 |
| EfficientDet D0 | 20.1 / 28.6 | 22.9 / 34.4 | 17.1 / 27.1 | 29.1 / 41.0 | 18.5 / 27.2 |
| EfficientDet D1 | 22.4 / 30.1 | 26.6 / 37.6 | 18.7 / 29.5 | 26.6 / 45.5 | 21.1 / 29.9 |
| SSD300 | 26.1 / 44.2 | 27.1 / 46.8 | 16.8 / 29.8 | 20.5 / 37.4 | 23.1 / 40.2 |
| CF-YOLO, K = 1 | 45.6 / 67.5 | 34.9 / 55.0 | 27.7 / 44.4 | 30.7 / 47.1 | 34.5 / 53.4 |
| CF-YOLO, K = 3 | 58.0 / 63.8 | 35.5 / 56.0 | 26.3 / 41.5 | 31.9 / 44.2 | 32.7 / 50.2 |

Table 1.5: Source: Table I of CF-YOLO.

For Table 1.5, as expected, performance decreases as snow becomes more difficult

For CF-YOLO K = 1:

- AP decreases from 45.6 on easy samples to 27.7 on difficult samples

- AP50 decreases from 67.5 to 44.4

- The AP50 drop is 23.1 percentage points

CF-YOLO performs strongly in several conditions, especially the easy level and particularly difficult AP. However, YOLOv7-tiny has the highest complete-RSOD result:

P=35.9, AP50=53.9

CF-YOLO K=1 obtains:

AP=34.5, AP50=53.4

1.7 Effect of Snow-Specific Training

| Method | Validation AP | Validation AP50 | Test AP | Test AP50 |
| --- | --- | --- | --- | --- |
| YOLOv5s | 34.2 -> 41.1 | 49.8 -> 62.5 | 32.7 -> 37.9 | 55.4 -> 64.6 |
| YOLOv6-nano | 32.6 -> 43.5 | 45.9 -> 60.2 | 36.4 -> 45.6 | 55.3 -> 66.3 |
| YOLOv7-tiny | 38.0 -> 44.3 | 53.5 -> 66.0 | 36.8 -> 40.9 | 56.3 -> 67.5 |
| EfficientDet D0 | 23.0 -> 28.9 | 33.3 -> 47.4 | 26.3 -> 33.3 | 40.0 -> 53.1 |
| EfficientDet D1 | 25.1 -> 37.8 | 34.5 -> 56.8 | 27.9 -> 40.2 | 41.6 -> 62.4 |
| SSD300 | 23.9 -> 34.7 | 42.4 -> 59.8 | 25.6 -> 33.1 | 46.8 -> 59.5 |
| CF-YOLO, K = 1 | 35.5 -> 41.2 | 51.8 -> 64.6 | 35.6 -> 42.4 | 57.6 -> 70.6 |
| CF-YOLO, K = 3 | 38.9 -> 47.5 | 56.5 -> 71.1 | 34.0 -> 41.9 | 58.4 -> 70.6 |

Table 1.6: Source: Table II of CF-YOLO.

For Table 1.6, the models are initialized using MSCOCO pretrained weights and trained on RSOD for 20 epochs. Each cell shows Before RSOD training -> After RSOD training.

CF-YOLO K=1 produces the best test AP of 42.4 and is tied with K=3 for the best test AP50 of 70.6.

After RSOD training:

| Metric | Improvement |
| --- | --- |
| Validation AP | +5.7 |
| Validation AP50 | +12.8 |
| Test AP | +6.8 |
| Test AP50 | +13.0 |

Table 1.7: CF-YOLO K=1 Improvement

| Metric | Improvement |
| --- | --- |
| Validation AP | +8.6 |
| Validation AP50 | +14.6 |
| Test AP | +7.9 |
| Test AP50 | +12.2 |

Table 1.8: CF-YOLO K=3 Improvement

Table 1.7 and Table 1.8 show that snow-specific training produces a large improvement; this effect not only appears on CF-YOLO as the results in Table 1.6.

This supports our decision to evaluate model components separately on GEN and SNOW instead of assuming that general-data importance is sufficient for snowy scenes.

1.8 General-Condition Accuracy and Efficiency

| Method | Parameters | GFLOPs | Val AP | Val AP50 | Test AP | Test AP50 | FPS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| YOLOv5s | 7.3 M | 17.3 | 36.3 | 55.3 | 36.6 | 55.3 | 65 |
| YOLOv6-nano | 4.3 M | 11.2 | 35.9 | 51.2 | – | – | 75 |
| YOLOv7-tiny | 6.2 M | 13.8 | 38.7 | 56.7 | 38.7 | – | 45 |
| EfficientDet D0 | 3.9 M | 2.5 | 34.3 | – | 33.8 | 55.2 | 63 |
| EfficientDet D1 | 6.6 M | 6.1 | 40.2 | – | 39.6 | 58.6 | 50 |
| CF-YOLO, K=1 | 9.2 M | 17.4 | 35.5 | 55.6 | 35.8 | 55.7 | 49 |
| CF-YOLO, K=3 | 22.0 M | 17.4 | 36.1 | 55.8 | 36.2 | 55.9 | 44 |

Table 1.9: Source: Table III of CF-YOLO.

For Table 1.9, the authors also train the models on MSCOCO for 300 epochs and measure speed using a Tesla V100 GPU.

Compared with YOLOv5s, CF-YOLO K = 1:

- increases parameters from 7.3 M to 9.2 M

- has nearly equal GFLOPs: 17.3 versus 17.4

- improves test AP50 by only 0.4

- reduces test AP by 0.8

- reduces speed from 65 FPS to 49 FPS

This means CF-YOLO K=1 has approximately 26% more parameters and approximately 25% lower FPS than YOLOv5s on the reported V100 test.

1.9 Short Summary

CF-YOLO is a peer-reviewed IEEE journal study focused on traffic-related object detection under snow.

The paper introduces RSOD, which contains 2,100 real snowy images and 7,862 annotated objects. RSOD is divided into easy, normal, difficult and particularly difficult levels according to snow coverage and human observation.

CF-YOLO modifies the YOLOv5s neck using Cross Fusion blocks and evaluates the model on both the general MSCOCO dataset and the snow-specific RSOD dataset.

Before snow-specific training, CF-YOLO K=1 achieves 34.5 AP and 53.4 AP50 across RSOD.

After 20 epochs of RSOD training, its test performance increases to 42.4 AP and 70.6 AP50.

However, CF-YOLO does not perform pruning: its K=1 model has 9.2 million parameters compared with 7.3 million for YOLOv5s, and its reported speed decreases from 65 FPS to 49 FPS.

1.10 Comparison with Us

| Item | CF-YOLO | Our project |
| --- | --- | --- |
| Publication status | Peer-reviewed IEEE journal | - |
| Base detector | YOLOv5s | YOLO26n |
| General dataset | MSCOCO | MIO-TCD |
| Snow dataset | RSOD | ACDC snow / Others |
| Snow images | 2,100 | 1,000~ / pending |
| Snow grading | Four difficulty levels | One snow domain |
| General evaluation | Yes | Yes |
| Snow evaluation | Yes | Yes |
| Optimization method | Cross Fusion neck | Weather-aware pruning |
| Fusion block | Yes | No |
| Layer sensitivity | No | GEN and SNOW sensitivity |
| Physical model pruning | No | Yes |
| Accuracy | Better | Lower |
| Model size | Bigger | Smaller |
| Model speed | Slower | Faster |
| Reported hardware | Tesla V100 | PYNQ-Z2 |
| Reported efficiency | Params, GFLOPs, FPS | Params, size, latency, FPS, memory, CPU usage, (can be added) |

Table 1.10: project comparation

Table 1.10 shows: the main difference is that we have custom pruning, while they have custom fusion block.

1.11 What could be useful from CF-YOLO

Snow severity evaluated separately

Although we did not separate snow levels, we separated snow and non-snow.

General and snow performance

CF-YOLO is tested on both MSCOCO and RSOD. This is similar to the use of GEN and SNOW datasets.

Snow-specific data significantly improve performance

Training on RSOD improves CF-YOLO test AP50 by approximately 12–13. This confirms that snow-specific data provides information that may not be learned from a general dataset.

Lightweight architecture is not equal to model compression

CF-YOLO improves snow detection through a redesigned feature-fusion neck, but it has more parameters and lower FPS than YOLOv5s.

2. Competitor: Traffic YOLO Pruning (WCE, BDD100K) [2]

2.1 Overview

It is a formally published journal article. It was accepted on 15 March 2026. The study focuses on the conflict between detection accuracy and computational cost in autonomous-driving systems.

Large YOLO detectors can provide strong object-detection performance, but their parameter counts and computation requirements make deployment difficult on resource-constrained onboard devices.

The authors argue that conventional pruning methods may be unsuitable for long-tailed traffic datasets. Traditional methods commonly rank channels according to weight magnitude. However, a channel with a small numerical magnitude may still contain important information.

To address this issue, the paper proposes a structured channel-pruning framework based on Weighted Copula Entropy, abbreviated as WCE. The method estimates the relationship between feature channels and object classes while increasing the importance of underrepresented classes.

2.2 Relevance

- It applies structured pruning to YOLO object detectors

- It uses BDD100K, a large autonomous-driving traffic dataset (including snow)

- It measures the effects of pruning on accuracy, FLOPs, parameters, and individual traffic classes

- Its pruning decision considers underrepresented information instead of relying only on weight magnitude.

The similarity is mainly in the pruning principle:

| Study | Information protected during pruning |
| --- | --- |
| WCE pruning | Features important to rare object classes |
| Our project | Features important to the SNOW domain as well as GEN |
| Conventional pruning | Mainly globally important or high-magnitude features |

Table 2.1: pruning

In table 2.1, the WCE study prevents the dominant car class from controlling the pruning decision.

Similarly, our project attempts to prevent general traffic data from controlling the pruning decision.

2.3 Main Contributions

2.3.1 Class aware channel pruning

The paper identifies a weakness in ordinary structured pruning: common classes produce more training samples and activations, so channels related to those classes tend to receive higher importance scores. Rare-class channels may therefore be removed even when they are safety-critical.

The proposed WCE method adds inverse class-frequency weighting so that rare classes contribute more strongly to channel evaluation.

2.3.2 Semantic importance

Instead of treating a large weight as automatically important, WCE measures the statistical dependency between a feature channel and the object detection targets.

A channel can therefore be retained because it contains useful semantic information, even when its weight or activation magnitude is relatively small.

2.3.3 Removal of redundant channels

After measuring channel relevance, the method uses maximum Relevance and Minimum Redundancy, or mRMR, to avoid retaining multiple channels that encode similar information.

The intended result is a smaller feature set that is:

- relevant to the detection classes

- protective of rare classes

- less similar function redundant

2.3.4 Physical structured pruning

The selected channels are physically removed from the model. This differs from unstructured, where zero-valued weights may remain inside the original tensors.

Structured channel removal can directly reduce:

- parameter count

- model memory

- FLOPs

- actual convolution dimensions

2.4 How the WCE Pruning Method Works

The framework can be simplified into four stages.

Step 1: Sparse training with Elastic Net

Elastic Net regularization is applied to batch-normalization scaling factors during sparse training. Its purpose is to push less useful channels toward small scaling values while maintaining a more stable sparse structure than using only L1 regularization. (combined both L1 and L2 prune)

Step 2: Weighted Copula Entropy evaluation

The feature maps produced by each channel are compared with the object-class labels.

Inverse class-frequency weights are applied so that a rare class receives greater importance than a frequent class during the information calculation.

Conceptually:

Rare class -> larger sample weight -> greater protection of its informative channels

Step 3: Weighted mRMR channel selection

The method selects channels with:

- similar function to detect classes

- low redundancy with already selected channels

Step 4: Iterative structured channel removal

Low-scoring channels are physically removed. The feature-selection and pruning process is performed iteratively until the target pruning rate is reached.

The complete framework therefore combines:

Structural sparsity + channel relevance + class balancing + redundancy removal

The paper describes this as a two-stage process consisting of sparse training followed by iterative channel selection and pruning.

2.5 BDD100K Dataset Usage

The study uses the BDD100K autonomous-driving dataset. According to the paper, BDD100K contains more than 100,000 driving videos and over 1,000 hours of driving footage. It covers different weather, lighting, urban/rural, daytime, and nighttime conditions.

The object-detection task contains ten traffic-object classes:

- Car

- Traffic Sign

- Traffic Light

- Person

- Truck

- Bus

- Bike

- Rider

- Motor

- Train

| Class | Object instances | Dataset share |
| --- | --- | --- |
| Car | 1,021,857 | 55.49% |
| Traffic Sign | 343,777 | 18.67% |
| Traffic Light | 265,906 | 14.44% |
| Person | 129,262 | 7.02% |
| Truck | 42,963 | 2.33% |
| Bus | 16,505 | 0.90% |
| Bike | 10,229 | 0.56% |
| Rider | 6,461 | 0.35% |
| Motor | 4,296 | 0.23% |
| Train | 179 | 0.01% |
| Total | 1,841,435 | 100% |

Table 2.2: Class distribution

In Table 2.2, car alone accounts for more than half of all annotations, while the Train class contains only 179 examples. The paper gives the more precise Train proportion as approximately 0.0097%.

2.5.1 Relevance

The dataset distribution demonstrates why aggregate accuracy can hide weak performance on a smaller but important group.

For the WCE paper:

High overall mAP (not imply) good rare-class detection

For our project:

High GEN mAP (not imply) good SNOW detection

This analogy supports the decision to measure pruning effects separately instead of using only one overall validation score.

2.6 Experimental Configuration

| Item | Paper configuration |
| --- | --- |
| Dataset | BDD100K |
| Models | YOLOv5l and YOLOv8l |
| Pruning type | Structured channel pruning |
| Main pruning rate | 50% |
| Comparison methods | L1, HRank, FPGM, LAMP |
| Main accuracy metric | mAP@0.5 |
| Compression metrics | FLOPs and parameters |
| Independent runs | Five random seeds |
| Programming language | Python 3.9.18 |
| Framework | PyTorch 2.0.1 |
| CUDA | 12.1 |
| cuDNN | 8.9.2 |
| CPU | Intel Xeon Silver 4116 |
| GPU | NVIDIA RTX8000 |
| Memory | 128 GB |

Table 2.3: setups

For Table 2.3, the paper states that identical hyperparameters and experimental settings were used across compared pruning methods.

However, the experimental section does not clearly provide key training details such as batch size, learning rate, number of epochs, or input resolution.

2.6.1 WCE selection overhead

| Model | Channel-selection time |
| --- | --- |
| YOLOv5l | 28 minutes |
| YOLOv8l | 32 minutes |

Table 2.4: WCE-mRMR channel-selection process takes approximately

For Table 2.4, the authors state that this one-time offline search contributes less than 5% of the complete training pipeline. It is performed before deployment and does not directly add computation to normal inference.

2.7 Overall Pruning Results

| Method | mAP@0.5 | FLOPs | Parameters |
| --- | --- | --- | --- |
| Baseline | 55.96 ± 0.21 | 165.22 G | 43.71 M |
| L1 | 53.82 ± 0.37 | 98.73 G | 25.92 M |
| HRank | 54.95 ± 0.30 | 92.09 G | 24.53 M |
| FPGM | 55.12 ± 0.32 | 89.28 G | 23.76 M |
| LAMP | 55.45 ± 0.27 | 95.36 G | 25.33 M |
| WCE | 55.82 ± 0.18 | 86.50 G | 22.87 M |

Table 2.5: YOLOv8l results

In table 2.5: relative to the baseline, WCE:

- reduces mAP@0.5 by only 0.14%

- reduces FLOPs by approximately 47.65%

- reduces parameters by approximately 47.68%

It also obtains higher mAP and lower model complexity than all four pruning baselines at the reported setting.

| Method | mAP@0.5 | FLOPs | Parameters |
| --- | --- | --- | --- |
| Baseline | 54.27 ± 0.24 | 108.12 G | 46.52 M |
| L1 | 52.15 ± 0.39 | 56.94 G | 23.89 M |
| HRank | 53.45 ± 0.35 | 55.67 G | 23.68 M |
| FPGM | 53.68 ± 0.29 | 54.63 G | 23.27 M |
| LAMP | 53.92 ± 0.28 | 55.36 G | 23.45 M |
| WCE | 54.18 ± 0.19 | 53.21 G | 22.14 M |

Table 2.6: YOLOv5l results

In table 2.6: Relative to the baseline, WCE:

- reduces mAP@0.5 by only 0.09%;

- reduces FLOPs by approximately 50.79%

- reduces parameters by approximately 52.41%

2.7.1 interpretation

The main result is the model should be smaller. It produces nearly half the original model complexity while retaining almost all overall mAP@0.5.

The reported standard deviations are also relatively small, and each result is based on five independent random-seed runs.

2.8 Class-Wise Results

The following table compares the unpruned baseline with the complete WCE-pruned model.

| Class | YOLOv8l Baseline -> WCE | Change | YOLOv5l Baseline -> WCE | Change |
| --- | --- | --- | --- | --- |
| Car | 80.25 -> 79.58 | −0.67 | 79.47 -> 78.85 | −0.62 |
| Traffic Sign | 70.18 -> 69.49 | −0.69 | 69.29 -> 67.95 | −1.34 |
| Traffic Light | 69.36 -> 68.67 | −0.69 | 68.50 -> 67.33 | −1.17 |
| Person | 66.02 -> 65.33 | −0.69 | 65.22 -> 63.87 | −1.35 |
| Truck | 59.87 -> 59.18 | −0.69 | 59.25 -> 58.22 | −1.03 |
| Bus | 57.63 -> 58.05 | +0.42 | 56.18 -> 56.31 | +0.13 |
| Bike | 52.89 -> 53.22 | +0.33 | 51.01 -> 51.11 | +0.10 |
| Rider | 49.45 -> 49.81 | +0.36 | 47.72 -> 47.77 | +0.05 |
| Motor | 48.26 -> 48.65 | +0.39 | 46.50 -> 46.55 | +0.05 |
| Train | 2.76 -> 5.12 | +2.36 | 0.00 -> 3.84 | +3.84 |

Table 2.7: Accuracy for each class

In Table 2.7, the results show two different patterns:

- Frequent classes such as Car, Traffic Sign, Traffic Light, Person, and Truck experience accuracy reductions.

- Several minority classes, including Bus, Bike, Rider, Motor, and Train, improve after pruning.

The most significant change occurs for Train:

YOLOv8l: 2.76->5.12

YOLOv5l: 0.00->3.84

This suggests that the class-balanced criterion retains channels that are useful to rare classes but may damage common classes.

2.9 Results under Different Weather Conditions

BDD100K includes a mixture of weather, lighting, urban/rural, daytime, and nighttime conditions. However, this paper does not provide separate results for each weather or visibility level.

Its reported numerical results are:

- aggregate BDD100K mAP@0.5

- class-wise mAP

- FLOPs

- parameters

2.10 Ablation Study

The authors test four versions of their framework.

| Model | Variant | mAP@0.5 | FLOPs | Parameters |
| --- | --- | --- | --- | --- |
| YOLOv5l | Elastic Net only | 52.86 | 54.12 G | 22.51 M |
|  | WCE without class balancing | 53.51 | 53.78 G | 22.32 M |
|  | WCE + class balancing, without mRMR | 53.97 | 53.37 G | 22.16 M |
|  | Complete WCE framework | 54.18 | 53.23 G | 22.07 M |
| YOLOv8l | Elastic Net only | 54.53 | 89.73 G | 23.52 M |
|  | WCE without class balancing | 55.07 | 88.24 G | 23.09 M |
|  | WCE + class balancing, without mRMR | 55.59 | 87.09 G | 22.68 M |
|  | Complete WCE framework | 55.82 | 86.47 G | 22.86 M |

Table 2.8: frameworks

In Table 2.8, the results show the role of each component:

- Elastic Net pushes less useful channels toward small scaling

- WCE map channel effective to classes

- Class balancing improves adaptation to rare classes

- mRMR removes redundant features

Compared with Elastic Net only, the complete framework improves mAP@0.5 by:

- 54.18 - 52.86 = 1.32 for YOLOv5l

- 55.82-54.53=1.29 for YOLOv8l

The complete framework also has the lowest FLOPs among the four variants.

2.10.1 Fusion-weight sensitivity

The method combines the normalized Elastic Net score and WCE-mRMR score using a balance coefficient α.

The authors test:

α = 0.0, 0.1, 0.2, …, 1.0

Both YOLOv5l and YOLOv8l reach their highest mAP@0.5 at:

α = 0.5

This means an equal balance between L1 L2 prune and proposed prune performs better than allowing either criterion to dominate the pruning decision.

This is relevant to our project because its GEN and SNOW sensitivity components should also be balanced carefully.

2.11 Short Summary

It applies structured channel pruning to YOLO-based autonomous-driving detectors using the BDD100K traffic dataset.

The proposed method combines Elastic Net sparse training, class-balanced Weighted Copula Entropy, and maximum-relevance minimum-redundancy channel selection.

Its objective is to prevent pruning from removing channels that are important to underrepresented traffic classes.

At a 50% pruning rate, the method reduces YOLOv8l from 43.71 million to 22.87 million parameters and from 165.22 to 86.50 GFLOPs, while mAP@0.5 decreases by only 0.14. For YOLOv5l, the model is reduced from 46.52 million to 22.14 million parameters and from 108.12 to 53.21 GFLOPs, with only a 0.09 mAP@0.5 reduction.

The method also improves the rare Train-class accuracy from 2.76 to 5.12 for YOLOv8l and from 0 to 3.84 for YOLOv5l.

However, the paper does not report weather-specific mAP, snow-condition performance, mAP50–95, inference latency, FPS, or embedded-device deployment.

Therefore, it provides a strong traffic-pruning competitor, while our project extends balanced pruning from class-aware channel selection to GEN- and SNOW-aware YOLO26n pruning for PYNQ-Z2 deployment.

2.12 Comparison with Us

| Item | WCE paper | Our project |
| --- | --- | --- |
| Publication status | Peer-reviewed journal | - |
| Detector | YOLOv5l and YOLOv8l | YOLO26n |
| General dataset | BDD100K | MIO-TCD |
| Snow dataset | None separately | ACDC Snow / (could be BDD100K) |
| Main imbalance | Frequent vs rare classes | GEN vs SNOW domains |
| Pruning unit | Channels (convolution) | Layers / channels / individual weights |
| Importance criterion | Elastic Net + WCE-mRMR | Measured GEN/SNOW accuracy drops |
| Main accuracy metric | mAP@0.5 | mAP50–95 |
| Parameter reduction | Approximately 48–52% | - |
| FLOP reduction | Approximately 48–51% | - |
| Per-weather evaluation | No | GEN and SNOW separately |
| Latency/FPS | Not reported | Measure on PYNQ-Z2 |
| Deployment hardware | RTX8000 | PYNQ-Z2 ARM |
| Main objective | Protect rare classes during pruning | Protect snow performance during pruning (while not ignoring up general) |

Table 2.9: project comparation

Table 2.9 shows our differences.

2.13 What could be useful from WCE

A small group should not be hidden by an aggregate metric

The paper demonstrates that a model can have good overall mAP while performing poorly on a rare but important traffic class.

For the current project, snow images represent a specialized condition that may similarly be hidden by overall GEN accuracy.

Importance should reflect task performance

WCE shows that weight magnitude alone may not identify whether a channel contains important semantic information.

Our framework takes this principle further by directly measuring the effect of removing a candidate on GEN and SNOW mAP50–95.

Balanced sensitivity can protect specialized features

WCE protects minority-class features through inverse-frequency weighting.

Our framework protects snow-sensitive components through separate SNOW evaluation and normalized accuracy drop.

Structured pruning validates on hardware

The WCE paper reports strong theoretical compression but not runtime measurements.

Our project can provide additional deployment evidence by comparing:

- NCNN latency

- FPS

- memory use

- model size

- PYNQ CPU usage

3. Negative AD Reasoning

3.1 Removal of Domain-Sensitive or Harmful Features (CVPR) [3]

Wang et al. showed that some feature channels learned from the source domain are not universally useful. A channel may improve performance on the original source domain but reduce performance after a domain shift.

Their channel-removal experiment used a detector trained on Cityscapes and evaluated it both on Cityscapes and the corrupted Cityscapes-C dataset. Some channels caused an in-domain accuracy reduction when removed but simultaneously produced a cross-domain accuracy improvement.

This finding provides a particularly relevant explanation for a negative AD in our project. For example, a pruned channel or pruning group may primarily encode general-weather textures, contrast patterns, backgrounds, or object appearances that are unreliable in snow. Removing that group can reduce misleading activations and improve mAP even though parameters have been removed. Such a group should not automatically be described as “unimportant”; it may instead be domain-sensitive (e.g. only useful on traffic, or snow, or pencils, etc.).

| Experiment | Direct test | Proposed method | Relevant result |
| --- | --- | --- | --- |
| Cityscapes -> Cityscapes-C, average mAP@50 | 4.5 | 11.4 | +6.9 mAP |
| Cityscapes-C, snow, round 1 | 0.6 | 2.6 | +2.0 mAP |
| Cityscapes-C, snow, round 5 | 0.6 | 4.6 | +4.0 mAP |
| Cityscapes-C, snow, round 10 | 0.6 | 4.8 | +4.2 mAP |
| Cityscapes -> ACDC, average mAP@50 | 22.0 | 24.2 | +2.2 mAP |
| ACDC snow, rounds 1 / 5 / 10 | 21.8 / 21.8 / 21.8 | 23.4 / 24.7 / 24.5 | Consistent snow improvement |

Table 3.1: suppress channels that changed strongly between domains & retaining invariant channels

For Table 3.1, it quantified channel sensitivity using both image-level and instance-level feature discrepancies between the source and target domains. It then applied sensitivity-weighted sparsity regularization to suppress channels that changed strongly between domains while retaining more invariant channels. The method used Batch Normalization scaling factors to control and eventually prune individual channels.

On Cityscapes-C, the proposed method obtained an average mAP@50 of 11.4 with 449.6 GFLOPs.

On ACDC, it obtained 24.2 average mAP with 452.8 GFLOPs. These results show that removing domain-sensitive channels can improve adverse-condition performance while also reducing computation.

| Configuration | Average mAP@50 | FLOPs reduction |
| --- | --- | --- |
| Direct test | 4.5 | 0.0% |
| Adaptation only | 10.9 | 0.0% |
| + indiscriminate sparsity pruning | 8.8 | 10.2% |
| + image-level sensitivity | 10.5 | 10.5% |
| + instance-level sensitivity | 9.9 | 10.9% |
| + both sensitivity measures | 11.2 | 10.7% |
| + stochastic channel reactivation | 11.4 | 10.4% |

Table 3.2: ablation study

Indiscriminate pruning reduced the adapted result from 10.9 to 8.8 mAP. Adding image-level and instance-level sensitivity guidance raised it to 11.2, and stochastic channel reactivation further increased it to 11.4.

3.2 Pruning as Regularization and Noise Injection (GSTNNP) [4]

Bartoldson et al. explained another possible cause of negative AD: pruning can act as a form of regularization rather than only as model compression.

When filters are removed, the information passed through the network changes. Therefore, the model may initially experience an accuracy drop. But the remaining filters may adapt to this change and learn to perform the task with fewer parameters.

The authors describe this effect as being similar to noise injection regularization. Adding noise or temporarily disturbing a model can prevent it from relying too strongly on specific features. As a result, the model may reduce overfitting and perform better on unseen validation or test data.

The paper uses pruning instability to measure the immediate effect of pruning:

Pruning Instability=Accbefore pruning-Accimmediately after pruningAccbefore pruning

A larger value means that pruning caused a larger immediate accuracy decrease.

For example, suppose the accuracy is 90% before pruning and 81% immediately after pruning:

Pruning Instability=90-8190=0.10

Therefore, the pruning instability is 10%.

The study found that a moderate immediate accuracy reduction could sometimes lead to better final accuracy after continued training. This means that the temporary disturbance caused by pruning may encourage the model to find a simpler and more generalizable solution.

The authors also tested temporarily setting selected filters to zero instead of permanently removing them. This produced a similar generalization effect to permanent pruning. Applying Gaussian noise for an extended training period also improved the final performance.

These results suggest that part of the accuracy improvement came from the disturbance-and-recovery process, rather than only from reducing the number of parameters.

4. Negative AD Handling

I suggest we retain negative values.

4.1 Support 1: Neuron Structured Pruning Polarization Regularizer (NSPPR) [5]

Zhuang et al. provide direct experimental evidence that negative AD can be a valid pruning outcome.

Their paper defines negative accuracy drop as a pruned model achieving higher accuracy than its unpruned baseline. Their reported pruned accuracy was measured after fine-tuning the pruned network.

| Dataset | Model | Baseline accuracy | Pruned accuracy | Accuracy drop | FLOPs reduction |
| --- | --- | --- | --- | --- | --- |
| CIFAR-10 | ResNet-56 | 93.80% | 93.83% | −0.03% | 47% |
| CIFAR-10 | VGG-16 | 93.88% | 93.92% | −0.04% | 54% |
| CIFAR-100 | VGG-16 | 73.83% | 74.25% | −0.42% | 43% |

Table 4.1: relevant negative-AD results

Other than Table 4.1, a previous DCP-adapt result in the same comparison also achieved -0.01% AD at 47% FLOPs reduction, while Network Slimming produced -0.37% AD for VGG-16 on CIFAR-100 (Their table: Results on CIFAR-10 and CIFAR-100. Best results are bolded).

Negative AD was therefore not unique to one method.

The polarization study also found that pruning amount and accuracy drop were not always monotonically related. Increasing its threshold parameter generally removed more FLOPs, but the resulting accuracy drop fluctuated.

4.2 Support 2: Learning Filter Pruning Criteria (LFPC) [6]

He et al. provided further evidence that negative AD can be retained as a valid pruning result.

The authors state that a negative value indicates improved model accuracy. Therefore, the negative values were directly reported.

The LFPC method produced negative AD results for different ResNet models on CIFAR-10:

| Dataset | Model | Baseline accuracy | Pruned accuracy | Accuracy drop | FLOPs reduction |
| --- | --- | --- | --- | --- | --- |
| CIFAR-10 | ResNet-56 | 93.59% | 93.72% | −0.13% | 47.1% |
| CIFAR-10 | ResNet-110 | 93.68% | 93.79% | −0.11% | 60.3% |

Table 4.2: Relevant negative AD results from LFPC

For Table 4.2, ResNet-56 pruning increased the accuracy from 93.59% to 93.72%.

Similarly, the pruned ResNet-110 achieved 93.79% accuracy compared with the baseline accuracy of 93.68%, resulting in an AD of -0.11% The paper retained both negative values and interpreted them as improvements in model accuracy.

These results demonstrate that negative AD can remain valid even when substantial computation is removed. Therefore, LFPC directly supports preserving the original sign of negative AD values.

4.3 Support 3: Filter Pruning via Geometric Median (FPGM) [7]

He et al. also retained negative AD values in their FPGM pruning experiments.

FPGM removes filters that are considered redundant according to their distance from the geometric median of the filters. The experimental results show that removing redundant filters can reduce computation while maintaining or slightly improving model accuracy.

The paper reported several negative accuracy-drop results for ResNet models on CIFAR-10:

| Dataset | Model and pruning setting | Baseline accuracy | Pruned accuracy | Accuracy drop | FLOPs reduction |
| --- | --- | --- | --- | --- | --- |
| CIFAR-10 | ResNet-32, FPGM-mix 40% | 92.63% | 92.82% | −0.19% | 53.2% |
| CIFAR-10 | ResNet-110, FPGM-only 40% | 93.68% | 93.73% | −0.05% | 52.3% |
| CIFAR-10 | ResNet-110, FPGM-mix 40% | 93.68% | 93.85% | −0.17% | 52.3% |
| CIFAR-10 | ResNet-110, FPGM-only 40% with fine-tuning | 93.68% | 93.74% | −0.16% | 52.3% |

Table 4.3: Relevant negative AD results from FPGM

In Table 4.3, FPGM-mix reduced 53.2% of the FLOPs of ResNet-32 while increasing its accuracy from 92.63% to 92.82%:

The authors described this result as a 0.19% accuracy improvement. For the fine-tuned ResNet-110 experiment, FPGM achieved a 52.3% FLOPs reduction and increased accuracy from 93.68% to 93.74%. The corresponding -0.16% AD was described as a performance increase.

Therefore, FPGM provides another example in which negative AD values were preserved in the reported results.

4.4 Support 4: Visual-Saliency-Guided Channel Pruning [8]

Choi and Tian provided direct YOLO-family evidence that a pruned object detector can outperform its unpruned baseline.

Their study applied structured channel pruning to YOLOX-S and evaluated the resulting models on the KITTI traffic dataset. The work was published in the 2023 IEEE Intelligent Vehicles Symposium.

The unpruned YOLOX-S achieved an mAP of 87.3%. At a pruning rate of 0.3, several pruning methods produced higher mAP values than the original model:

| Dataset | Model | Parameters | FLOPs | mAP | Calculated AD |
| --- | --- | --- | --- | --- | --- |
| KITTI | Vanilla YOLOX-S | 8.94 M | 26.65 G | 87.3% | 0.0% |
| KITTI | L1-pruned YOLOX-S | 5.40 M | 18.71 G | 88.5% | −1.2% |
| KITTI | CHIP-pruned YOLOX-S | 5.35 M | 18.88 G | 88.9% | −1.6% |
| KITTI | Proposed pruned YOLOX-S | 5.35 M | 18.88 G | 89.1% | −1.8% |

Table 4.4: Relevant negative AD results for pruned YOLOX-S

In thble 4.4, the mAP dropped by -1.8%, at the same time, the parameters were reduced from 8.94 M to 5.35 M. The FLOPs were reduced from 26.65 G to 18.88 G.

The authors described this result as a 1.8% mAP improvement over the unpruned model with 40.2% of the parameters and 29.2% of the FLOPs removed. Therefore, the result was treated as a genuine performance improvement rather than being converted into an absolute accuracy loss or replaced with zero.

The comparison also shows that negative AD was not limited to the proposed saliency-guided method. L1 pruning and CHIP produced calculated AD values of -1.2% and -1.6%, respectively. This provides additional evidence that a higher post-pruning mAP can occur across different pruning criteria.

4.5 Support 5: Streamlining YOLOv7 for Rapid and Accurate Detection [9]

Gu et al. presented another direct example of a negative AD result using YOLOv7.

The study developed a dual-dimensional pruning method that removes redundant structures in both the spatial and channel dimensions. Three channel-pruning strategies were evaluated, and the authors selected custom-ratio layer-by-layer pruning as the best-performing configuration. The study was published in Sensors in 2024.

The final pruned YOLOv7 model achieved a slightly higher mAP than the original YOLOv7 baseline:

| Dataset/task | Model | Parameters | Inference time (Raspberry Pi 4B) | mAP | Calculated AD |
| --- | --- | --- | --- | --- | --- |
| Rapeseed variety detection | Original YOLOv7 | 36.50 M | 4.48 s/image | 96.68% | 0.0% |
| Rapeseed variety detection | Custom1 pruned YOLOv7 | 9.19 M | 1.18 s/image | 96.89% | −0.21% |

Table 4.5: Relevant negative AD result for pruned YOLOv7

The authors stated that the Custom1 model’s mAP surpassed the baseline model. Therefore, the higher post-pruning accuracy was retained as an improvement.

This result is relevant to the current project because the pruned model was tested on a resource-constrained embedded platform. It demonstrates that a YOLO model can achieve a smaller parameter count, shorter inference time, and slightly higher detection accuracy after carefully controlled pruning.

4.6 Conclusion

| Support | Architecture | Negative result | Resource reduction | Handling |
| --- | --- | --- | --- | --- |
| Polarization Pruning [5] | ResNet/VGG | −0.03% to −0.42% | Up to 54% FLOPs | Retained as improved accuracy |
| LFPC [6] | ResNet | −0.11% to −0.13% | Up to 60.3% FLOPs | Retained as improved accuracy |
| FPGM [7] | ResNet | −0.05% to −0.19% | Up to 53.2% FLOPs | Retained as performance improvement |
| Visual-saliency pruning [8] | YOLOX-S | −1.2% to −1.8% | 40.2% parameters and 29.2% FLOPs | Reported as mAP improvement |
| Dual-dimensional pruning [9] | YOLOv7 | −0.21% | 74.8% parameters and 73.7% inference time | Reported as surpassing baseline |

Table 4.6

I suggest we retain negative values.

5. PYNQ-Z2 PS-Side Baseline Evaluation

5.1 Objective

This experiment measured the software-only inference baseline of YOLO26n on the PYNQ-Z2 processing system (PS).

Three NCNN models were evaluated:

- Baseline Official: the official YOLO26n model trained on the COCO dataset with 80 object classes.

- Baseline Full-Data: a YOLO26n model trained on approximately 88,000 MIO-TCD traffic images with 11 traffic-object classes.

- Baseline Reduced-Data: the same YOLO26n architecture trained on 7,700 MIO-TCD images, which is approximately 8.75% of the training data used by the full-data model.

The full-data and reduced-data models use the same network architecture and class definitions. Their NCNN parameter files are identical, while their weight files are different.

Therefore, the two models have the same computational structure but different learned weights. The term “Reduced-Data” refers to the smaller training dataset and does not indicate a smaller or compressed model architecture.

5.2 Experimental Setup

All models were executed using the NCNN CPU inference framework on the PYNQ-Z2 PS side. The board used a 32-bit ARMv7 environment with two logical CPU cores.

The experiments used an input resolution of 640 × 640 pixels and two NCNN CPU threads.

| Setting | Value |
| --- | --- |
| Input resolution | 640 × 640 |
| Batch size | 1 |
| CPU threads | 2 |
| Warm-up inferences | 5 |
| Benchmark images | 50 |
| Repetitions | 3 |
| Total measured inferences | 150 |
| Confidence threshold | 0.25 |
| NMS IoU threshold | 0.70 |
| Maximum detections | 300 |

Table 5.1: benchmark configurations

About Table 5.1, the benchmark configuration was kept identical for all three models.

The reported compute latency includes image preprocessing, NCNN inference, output decoding, and non-maximum suppression.

Model loading and warm-up execution were excluded.

The FPS value was calculated from the mean compute latency.

5.3 Results

| Model | Training data | Classes | NCNN size (MiB) | Median latency (ms/image) | FPS | Peak memory (MiB) | CPU usage (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline Full-Data | Approximately 88,000 MIO-TCD images | 11 | 9.189 | 10,586.52 | 0.09469 | 132.96 | 95.27 |
| Baseline Reduced-Data | 7,700 MIO-TCD images | 11 | 9.189 | 10,592.06 | 0.09462 | 133.03 | 95.28 |
| Baseline Official | COCO | 80 | 9.311 | 11,042.63 | 0.09076 | 142.82 | 94.92 |

Table 5.2: PYNQ-Z2 PS NCNN performance

In Table 2.2, the full-data and reduced-data models required approximately 10.6 seconds to process one image (a lot!), corresponding to approximately 0.095 FPS. The official model required approximately 11.0 seconds per image and achieved approximately 0.091 FPS.

5.4 Full-Data and Reduced-Data Comparison

The full-data and reduced-data models showed almost identical embedded inference performance. The reduced-data model had only approximately 0.05% higher median latency and approximately 0.08% lower FPS than the full-data model. Their deployment model sizes were also identical.

This is because reducing the number of training images changes the learned weights but does not reduce the number of layers, channels, parameters, or operations in the model.

The NCNN parameter files of the two models were identical, confirming that their network structures were unchanged. Their NCNN binary files were different, confirming that they contained different trained weights.

Therefore, training with fewer images should not be expected to produce a smaller or faster model. Its main expected effect is on detection accuracy and generalization.

5.5 Official Model Comparison

The official model was slightly larger and slower than the two MIO-TCD models. Compared with the full-data model, the official model had:

- approximately 1.33% larger NCNN model size

- approximately 4.31% higher median latency

- approximately 4.14% lower FPS

- approximately 7.41% higher peak memory usage

One likely reason is that the official model contains an 80-class COCO detection head, while the MIO-TCD models contain only 11 output classes. The official output tensor contains 84 features per candidate detection—four bounding-box values and 80 class scores—whereas the MIO-TCD models contain only 15 features per candidate detection.

5.6 Bottleneck Analysis

For all three models, the main bottleneck was the NCNN network inference rather than image loading or post-processing.

| Stage | Mean time |
| --- | --- |
| Image reading | 22.83 ms |
| Preprocessing | 73.82 ms |
| NCNN inference | 10,485.54 ms |
| Post-processing | 1.86 ms |

Table 5.3: full-data model mean time

According to Table 2.3, NCNN inference contributed approximately 99.28% of the compute time. The normalized CPU usage was also approximately 95%, showing that the two ARM CPU cores were already heavily utilized.

Therefore, optimizing only image reading, preprocessing, or NMS would not provide a major speed improvement. The main reduction must come from decreasing the neural-network computation.

5.7 Real-Time Requirement

The current PS-only results do not satisfy real-time traffic-detection requirements. The three models achieved only approximately 0.09 FPS, meaning that one new frame was processed every 10–11 seconds.

These results should be treated as the unoptimized software baseline. For example, compared with the current full-data baseline:

| Target | Required speed-up |
| --- | --- |
| 1 FPS | approximately 10.6× |
| 5 FPS | approximately 52.8× |
| 10 FPS | approximately 105.6× |

Table 5.4: targets for real time detection

For Table 2.4, this large performance gap can be reduced by the next project stages, including structured pruning, reduced model complexity, INT8 quantization, lower input resolution, etc.

5.8 Currently

The current results include deployment model size, latency, FPS, peak memory usage, and CPU usage. The following measurements are still pending (thinking about whether I should run those on PYNQ, because that will take a super long while to perform validation):

- mAP50–95

- mAP50

- Precision

- Recall

- per-class AP

- number of parameters

- FLOPs

- power consumption (this one, I am not sure how to do so. See if we need this)

5.9 NCNN interface

Since NCNN model inference accounts for more than 99% of the total processing time, the main optimization should target the neural-network computation itself.

To reduce the NCNN inference time (as we know):

- Reduce the input image resolution
Test smaller inputs such as 640×640 to 320×320, then compare accuracy and latency.

- Quantize and optimize the NCNN model
Convert the model from FP32 to INT8 and run ncnn-optimize to fuse and simplify supported operations.

- Prune and simplify the model
With our proposed formular.

- Optimize NCNN execution on the ARM processor
Compile NCNN in Release mode with ARM NEON enabled, and test both one-thread and two-thread inference on the PYNQ-Z2.

- Move expensive layers to the FPGA (pending)
Identify the convolution layers that consume most inference time and accelerate those layers using the FPGA programmable logic.

5.10 Conclusion

The PS-only NCNN evaluation showed that YOLO26n can be successfully executed on the PYNQ-Z2 ARM processor, but its current performance is not enough for real-time traffic detection.

The full-data and reduced-data models both achieved approximately 0.095 FPS because they use the same network architecture despite being trained with substantially different amounts of data. The official 80-class model was slightly slower and required more memory because of its larger detection output.

Overall, the results gave a software baseline and the need for actual model compression. Future experiments will compare these baseline results with pruned, quantized, etc. models using the same configuration.

6. Pruning: Layer ablation using parameter-free replacement

I tested whether complete YOLO26n layers could be removed. After removing a layer, we checked:

- how many parameters were saved

- how much accuracy was lost

- whether the modified model could still run

- whether fine-tuning could recover the lost accuracy

The experiment followed this workflow:

- Prepare datasets and baselines

- Remove one layer at a time

- Test combinations near the target reduction (10.25%)

- Fine-tune the best candidates

- Compare the recovered models

the original layer was replaced with a no-parameter adapter:

E.g. before pruning:

Layer 8 -> Layer 9 -> Layer 10

After pruning:

Layer 8 -> Adapter -> Layer 10

YOLO layers are connected to other layers through feature-map dimensions.

For example, a later layer may expect:

Input shape: 80 channels x 40 x 40

Deleting a layer incorrectly may produce:

64 channels x 40 x 40

This creates a dimension mismatch and the model stops with an error.

The no-parameter adapter preserves the required connection format. It does not contain trainable weights, so it does not restore the removed parameters.

6.1 Dataset and baseline preparation

The MIO-TCD and ACDC dataset was converted into YOLO format. The final fixed split contained:

| Split | MIO-TCD | ACDC snow |
| --- | --- | --- |
| Training | 88,000 | 400 |
| Validation | 11,000 | 100 |
| Test | 11,000 | 500 |

Table 6.1: data split

For table 6.1, the dataset split was frozen, so every later experiment used exactly the same data.

The original GEN and SNOW models were evaluated using the server environment. The baseline results were:

| Model | mAP50–95 | mAP50 |
| --- | --- | --- |
| GEN baseline | 0.6414 | 0.8300 |
| SNOW baseline | 0.1899 | 0.3241 |

Table 6.2: baseline mAP

For Table 6.2, these local results became the denominators for later Accuracy Drop calculations. (similar as Rafed’s result)

6.2 Remove one layer at a time

The main question was: What would happen if we remove only this layer?

The script inspected the YOLO26n model and identified layers containing parameters.

Layers without parameters were skipped because removing them would not reduce the model size.

A total of:

- 17 parameterized layers were tested

- 6 zero-parameter layers were skipped

For every tested layer:

- Load the original model

- Find the selected layer

- Record its parameter count

- Replace it with a no-parameter adapter

- Save the modified model

- Reload the model to confirm it was valid

- Evaluate it on GEN and SNOW

- Record its accuracy and parameter reduction

For each layer:

Accuracy Drop = Baseline mAP50–95 − Pruned-model mAP50–95

Layer 9, the SPPF layer, was the strongest individual candidate.

It gave approximately:

6.56% parameter reduction

Its raw accuracy changed as follows:

| Dataset | Baseline | After Layer 9 replacement |
| --- | --- | --- |
| GEN | 0.6414 | 0.5401 |
| SNOW | 0.1899 | 0.1558 |

Table 6.3: pruned layer 9

Table 6.3 shows that Layer 9 could be removed while preserving more accuracy than many other tested layers.

Other large layers, such as Layers 20 and 22, reduced more parameters but caused greater raw accuracy damage.

6.3 Test combinations of layers

The target was approximately 10.25% (5% of 10.25% var) parameter reduction.

Combinations were created using the previous results.

For each combination:

- Load the baseline checkpoint.

- Replace all selected layers with no-parameter adapters.

- Verify the model could run.

- Count the remaining parameters.

- Evaluate GEN.

- Evaluate SNOW.

- Save the results and model.

The best raw combination was:

C007 = Layers 9 and 19

This combination reduced approximately:

10.36% of the model parameters

Its raw accuracy was:

| Dataset | Baseline | Raw C007 |
| --- | --- | --- |
| GEN | 0.6414 | 0.0590 |
| SNOW | 0.1899 | 0.0761 |

Table 6.4: massive raw drop

Although the raw accuracy was low, C007 was still the strongest candidate among the combinations near the 10.25% target.

The candidate closest to exactly 10.25% was C001:

- Layers 0, 2 and 10

- Reduction: approximately 10.24%

However, its GEN and SNOW mAP50–95 values were both zero. Therefore, it was rejected.

I can conclude that the raw accuracy drop will be very high when trying to prune more than one whole layer at the same time.

A single Layer 9 replacement was kept as a single layer control model.

| Candidate | Removed layers | Purpose |
| --- | --- | --- |
| C007 | 9 and 19 | Param drop near target |
| L9 | 9 | Single-layer control |

Table 6.5: recovery sets

Table 6.5 shows the two candidates selected for recovery.

6.4 Fine-tune and recover the selected models

Removing layers damaged the pretrained feature processing. Fine-tuning could teach the remaining model to work without those layers. This process is called recovery training.

Each candidate was fine-tuned separately for GEN and SNOW.

The four recovery tasks were:

- C007 on SNOW

- L9 on SNOW

- C007 on GEN

- L9 on GEN

The main settings of fine-tuning were:

| Setting | Value |
| --- | --- |
| Epochs | 20 |
| Image size | 640 |
| Batch size | 16 |
| Optimizer | AdamW |
| Initial learning rate | 0.001 |
| Weight decay | 0.0005 |
| Warm-up | 1 epoch |
| AMP | Enabled |
| Pretrained reload | Disabled |

Table 6.6: settings

In table 6.6, pretrained=False did not mean that training started from a random model. The replacement checkpoint had already been loaded. It meant that Ultralytics should not load another pretrained model over the modified architecture.

6.3 Results Comparing (with Rafed)

Method A: Whole-Layer Replacement

This method removes the entire YOLO layer and replaces it with a simple no-parameter adapter.

Method B: Dependency-Group Channel Pruning

This method removes selected channels rather than removing an entire layer.

A fair comparison should use:

- Approximately the same parameter reduction

- The same GEN and SNOW datasets spilt

- The same image size and evaluation settings

- The same fine-tuning budget

- The final recovered models

Both C007 and T6 used approximately 10.25% parameter reduction and 20 epochs of recovery training.

Therefore, they provide the closest current comparison.

Both methods lost substantial accuracy immediately after pruning.

| Raw pruned model | GEN retention | SNOW retention |
| --- | --- | --- |
| Whole-layer L9 | 84.20% | 82.06% |
| Whole-layer C007 | 9.21% | 40.05% |
| Dependency-group T5 | 30.81% | 44.80% |

Table 6.7: raw pruning result

Table 6.7 shows the dependency-group model suffered less immediate GEN damage.

This is understandable because channel pruning preserves most of each layer, while whole-layer replacement completely removes the feature-processing operation of the selected layer.

(In this state, I thought Method B must be better, until I run the next step.)

However, both experiments also show that fine-tuning is essential. The final result should therefore be judged using C007 recovered and T6 recovered, not their raw pruned versions

The two experiments used slightly different measured baseline values. For this reason, accuracy retention is more suitable than comparing only the raw mAP values (how much of the original model’s accuracy remains after pruning and fine-tuning).

| Method | Parameters reduced | GEN mAP50–95 | GEN retention | SNOW mAP50–95 | SNOW retention |
| --- | --- | --- | --- | --- | --- |
| Whole-layer replacement, L9 | 6.56% | 0.6347 | 98.95% | 0.1908 | 100.47% |
| Whole-layer replacement, C007 | 10.36% | 0.6184 | 96.42% | 0.1807 | 95.14% |
| Dependency-group pruning, T6 | 10.25% | 0.6371 | 98.45% | 0.1818 | 94.72% |

Table 6.8: param & retention compare

In Table 6.8, C007 did slightly better on SNOW, while T6 did better on GEN.

The average retention across GEN and SNOW was:

- C007: approximately 95.78%

- T6: approximately 96.59%

Their results are close.

| Aspect | Whole-layer replacement | Dependency-group channel pruning |
| --- | --- | --- |
| Pruning unit | Complete top-level layer | Selected connected channels |
| Method complexity | Simpler pruning decision | More complex dependency management |
| Control of reduction | Difficult because layers have fixed sizes | Easier to adjust gradually |
| Immediate accuracy damage | Can be very large | Usually more controlled |
| Experience parameters reduced | More | Very close to C007 |
| Experience GEN result | Lower | Better |
| Experience SNOW result | Better | Very close to C007 |
| Model structure | Using a custom no-parameter adapter | Physically reduces channels in existing structures |

Table 6.9: Advantages and disadvantages

In table 6.9, no matter which method we use, the other method can be an additional experiment used to show what happens when a different design choice is used (ablation experiment).

Rafed’s report also describes whole-module experiments as useful diagnostic or competitor evidence, while presenting physical dependency-group pruning as the main deployable compression method.

Since their results are so close, I may want to do further comparison in PYNQ (mainly on latency) and also compare their actual model sizes. (I will ask Rafed for his pt model)

7. Pruning: Global L1 channel-magnitude (to show our final model is better)

(pending)

If time allows, I will use more models to show that our final model is better.

7.1 Settings and procedures

7.2 Results comparing

8. Formulars suggestions

Since three of our thoughts on the formula are quite different, I think we have two options.

8.1 Option 1: compare the results of different formulas

Initially, we got:

NADGEN,i=mAPGEN,baseline-mAPGEN,pruned,imAPGEN,baseline

NADSNOW,i=mAPSNOW,baseline-mAPSNOW,pruned,imAPSNOW,baseline

NADGEN,i≤0.05  and  NADSNOW,i≤0.05

Parameters%i=ParametersiTotal Model Parameters×100

Formula A:

Si=αNADGEN,i+(1-α)NADSNOW,i

Pi=Parameters%iSi

Formula B:

Snew,i=1NADGEN,i-NADSNOW,i

Pi=Parameters%iSi

Formula C:

Si=max⁡NADGEN,i NADGEN,i+NADSNOW,i2

Bi=0.5×RParameters,i+0.5×RGFLOPs,i

Pi=Bi1+Siτ2 , τ=0.05

For formula C, the completed equation can be explained as:

- rewards useful resource reduction while strongly penalizing groups that cause accuracy loss

The result of comparation can be our contribution, states that we tested the results on two equations.

The one with better outcomes will then be used, and the final model will depend on it.

8.2 Option 2: Ebrahim determines the final formula

It may take a long while for the three of us to reach a consensus. Letting one person make the final decision is the most efficient way.

9. Summary

9.1 Competitor: Snow Aware (CF YOLO)

- CF-YOLO improves snowy-condition detection by replacing the YOLOv5s neck with Cross Fusion blocks that combine features from multiple backbone levels.

- The RSOD dataset contains 2,100 real snow images and divides them into four difficulty levels, showing that detection accuracy generally decreases as snow severity increases.

- Snow-specific training produces large accuracy improvements, which supports evaluating and training the model separately on the GEN and SNOW datasets.

- Although CF-YOLO improves snow detection, it is not a compressed model because it has more parameters and lower FPS than the original YOLOv5s.

9.2 Competitor: Traffic YOLO Pruning (BDD100K)

- The WCE method performs structured channel pruning while protecting features that are important to rare traffic-object classes.

- At approximately 50% pruning, WCE removes nearly half of the parameters and FLOPs of YOLOv5l and YOLOv8l while causing only a small overall mAP@0.5 reduction.

- WCE also improves several minority classes, demonstrating that a good overall mAP does not guarantee good performance for smaller but important data groups.

- The WCE principle supports our GEN–SNOW framework because general traffic data should not dominate the pruning decision and hide damage to snow-related features.

9.3 Negative AD Reasoning & Handling

- Negative AD may occur when pruning removes redundant, overfitted, domain-sensitive, or harmful features from the model.

- Pruning can also act as regularization by temporarily disturbing the model and encouraging it to learn a simpler and more generalizable solution during fine-tuning.

- Multiple structured-pruning studies, including YOLOX-S and YOLOv7 experiments, retain negative AD values and interpret them as genuine accuracy improvements.

9.4 Whole-Layer Replacement vs Dependency-Group Channel Pruning

- Dependency-Group Channel Pruning wins on raw accuracy.

- Their results are very close after fine-tuning.

9.4 PYNQ-Z2 PS-Side Baseline Evaluation

- The PYNQ-Z2 PS-side baseline requires approximately 10–11 seconds per image and achieves only about 0.09 FPS, which is not sufficient for real-time traffic detection.

- The full-data and reduced-data models have almost identical latency because changing the amount of training data does not change the model architecture or computational cost.

- NCNN inference contributes more than 99% of the total processing time, so the main optimization must reduce neural-network computation rather than only improving preprocessing or post-processing.

9.5 Pruning

- The results of Dependency-Group Channel Pruning and Whole Layer Raplacement are close.

9.6 Our Formular

- The final prunability formula should balance GEN accuracy, SNOW accuracy, and parameter reduction. Competing formulas could be compared experimentally.

10. My overall notes

10.1 Competitor

- Evaluating every pruning candidate separately on GEN and SNOW: one overall accuracy value may hide poor snow performance.

- Snow-specific fine-tuning: the CF-YOLO results show that snow data can provide information that is not learned from general traffic data.

- Snow images could be divided into different severity levels in future experiments if enough labelled data are available.

- Per-class accuracy could be reported because overall mAP may hide damage or improvements for smaller traffic-object classes.

10.2 Accuracy Drop

- Raw negative AD values should retain their original signs because they represent measured improvements rather than accuracy losses.

- The competing prunability formulas could be tested with the pruning results. (may suggest more)

10.3 Model

- We could define clear targets for acceptable GEN loss, acceptable SNOW loss, model size, latency, and FPS before going for the final model.

- NCNN optimization could be focus on lower input resolutions, INT8 quantization, ncnn-optimize, ARM NEON support, and appropriate thread settings.

- The most computationally expensive convolution layers could be considered for FPGA acceleration after layer-wise profiling. (in future)

- Currently our contribution will emphasize GEN–SNOW-aware structured pruning of YOLO26n together with measured PYNQ-Z2 deployment results.

10.4 Extra

1.11 May learn from CF-YOLO

2.13 May learn from BDD100K

4. I suggest we retain negative values

5. FPS is low, may need aggressive pruning

6.2 pruning compare

7. formula: if we do not have a consensus, we may do experience and that could also be our contribution.

Finally: This report is long, I may make mistakes, I may miss something to summarize, you may correct me or make comments in such cases. Thx you for spending time on reading the whole report.

References

[1] Ding, Q., Li, P., Yan, X., Shi, D., Liang, L., Wang, W., Xie, H., Li, J., & Wei, M. (2023). CF-YOLO: Cross fusion YOLO for object detection in adverse weather with a high-quality real snow dataset. IEEE Transactions on Intelligent Transportation Systems, 24(10), 10749–10759. https://doi.org/10.1109/TITS.2023.3285035

[2] Zhou, Y., Ma, J., & Dong, H. (2026). Weighted copula entropy for structural pruning in long-tailed autonomous driving object detection. Entropy, 28(3), Article 336. https://doi.org/10.3390/e28030336

[3] Wang, K., Fu, X., Lu, X., Ge, C., Cao, C., Zhai, W., & Zha, Z.-J. (2025). Efficient test-time adaptive object detection via sensitivity-guided pruning. In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition (pp. 10577–10586). https://doi.org/10.1109/CVPR52734.2025.00989

[4] Bartoldson, B. R., Morcos, A. S., Barbu, A., & Erlebacher, G. (2020). The generalization-stability tradeoff in neural network pruning. Advances in Neural Information Processing Systems, 33. https://proceedings.neurips.cc/paper/2020/hash/ef2ee09ea9551de88bc11fd7eeea93b0-Abstract.html

[5] Zhuang, T., Zhang, Z., Huang, Y., Zeng, X., Shuang, K., & Li, X. (2020). Neuron-level structured pruning using polarization regularizer. Advances in Neural Information Processing Systems, 33. https://proceedings.neurips.cc/paper/2020/hash/703957b6dd9e3a7980e040bee50ded65-Abstract.html

[6] He, Y., Ding, Y., Liu, P., Zhu, L., Zhang, H., & Yang, Y. (2020). Learning filter pruning criteria for deep convolutional neural networks acceleration. In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition (pp. 2009–2018). https://openaccess.thecvf.com/content_CVPR_2020/html/He_Learning_Filter_Pruning_Criteria_for_Deep_Convolutional_Neural_Networks_Acceleration_CVPR_2020_paper.html

[7] He, Y., Liu, P., Wang, Z., Hu, Z., & Yang, Y. (2019). Filter pruning via geometric median for deep convolutional neural networks acceleration. In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition (pp. 4340–4349). https://doi.org/10.1109/CVPR.2019.00447

[8] Choi, J. I., & Tian, Q. (2023). Visual-saliency-guided channel pruning for deep visual detectors in autonomous driving. In Proceedings of the 2023 IEEE Intelligent Vehicles Symposium (IV) (pp. 1–6). https://doi.org/10.1109/IV55152.2023.10186819

[9] Gu, S., Meng, W., & Sun, G. (2024). Streamlining YOLOv7 for rapid and accurate detection of rapeseed varieties on embedded device. Sensors, 24(17), Article 5585. https://doi.org/10.3390/s24175585