# Week 10: 27-31 July 2026

Table of Content

| Content | Page |
| --- | --- |
| 1. Competitor: Snow Aware (CF YOLO) | 2 – 3 |
| 2. Competitor: Traffic YOLO Pruning (BDD100K) | 4 – 5 |
| 3. Negative AD Reasoning (with paper support) | 6 |
| 4. Negative AD Handling (with paper support) | 7 – 8 |
| 5. PYNQ-Z2 PS-Side Baseline Evaluation | 9 – 10 |
| 6. Pruning: Layer Ablation vs Dependency Channel | 11 – 14 |
| 7. Our Formular | 15 |
| 8. Summary | 16 |
| References | 17 |

→

1. Competitor: Snow Aware (CF YOLO) [1]

1.1 Main Contributions of CF-YOLO

RSOD Dataset

The authors establish RSOD: Real-world Snowy Object Detection Dataset, containing 2,100 real snow images. Most images show townscape and traffic scenes, and annotations are supplied in both COCO and YOLO formats.

Snow Coverage Rate

The paper defines the Snow Coverage Rate, or SCR:

SCR=AsnowAbbox

where:

- Asnow is the estimated snow-covered area inside an object;

- Abbox is the area of the object bounding box.

SCR is combined with human observation to divide RSOD into four difficulty levels.

Cross Fusion Block

CF-YOLO replaces the FPN and PANet-based neck of YOLOv5s with Cross Fusion blocks.

The CF block directly combines features from several backbone stages.

The intention is to recover low-level information such as edges, outlines and texture that may be weakened or hidden by snow.

CF-YOLO uses two stacked CF blocks, while the convolution kernel size K can be set to 1 or 3.

K=1 focuses on cross-level feature fusion using 1×1 kernels, while K=3 uses 3×3 kernels to capture additional information.

1.2 Snow Difficulty Levels

| Snow difficulty | Images | Dataset share | General description |
| --- | --- | --- | --- |
| Easy | 600 | 28.6% | Objects remain relatively clear |
| Normal | 1,000 | 47.6% | Noticeable snow degradation |
| Difficult | 400 | 19.0% | Substantial snow coverage |
| Particularly difficult | 100 | 4.8% | Severe snow coverage |
| Total | 2,100 | 100% |  |

Table 1.3: RSOD four levels

For Table 1.3, the paper indicates that difficult samples generally have SCR between 0.5 and 0.7, while particularly difficult samples generally have SCR greater than 0.7. Difficulty is mainly assigned through human observation, with SCR used when the level is uncertain.

It demonstrates that treating all snow images as one condition may hide differences between light and severe snow. (this could be the reason we separate anow and non-snow)

1.3 Effect of Snow-Specific Training

| Method | Validation AP | Validation AP50 | Test AP | Test AP50 |
| --- | --- | --- | --- | --- |
| YOLOv5s | 34.2 -> 41.1 | 49.8 -> 62.5 | 32.7 -> 37.9 | 55.4 -> 64.6 |
| YOLOv6-nano | 32.6 -> 43.5 | 45.9 -> 60.2 | 36.4 -> 45.6 | 55.3 -> 66.3 |
| YOLOv7-tiny | 38.0 -> 44.3 | 53.5 -> 66.0 | 36.8 -> 40.9 | 56.3 -> 67.5 |
| EfficientDet D0 | 23.0 -> 28.9 | 33.3 -> 47.4 | 26.3 -> 33.3 | 40.0 -> 53.1 |
| EfficientDet D1 | 25.1 -> 37.8 | 34.5 -> 56.8 | 27.9 -> 40.2 | 41.6 -> 62.4 |
| SSD300 | 23.9 -> 34.7 | 42.4 -> 59.8 | 25.6 -> 33.1 | 46.8 -> 59.5 |
| CF-YOLO, K = 1 | 35.5 -> 41.2 | 51.8 -> 64.6 | 35.6 -> 42.4 | 57.6 -> 70.6 |
| CF-YOLO, K = 3 | 38.9 -> 47.5 | 56.5 -> 71.1 | 34.0 -> 41.9 | 58.4 -> 70.6 |

Table 1.6: Source: Table II of CF-YOLO.

For Table 1.6, the models are initialized using MSCOCO pretrained weights and trained on RSOD for 20 epochs. Each cell shows Before RSOD training -> After RSOD training, they are all improved.

This supports our decision to evaluate model components separately on GEN and SNOW instead of assuming that general-data importance is sufficient for snowy scenes.

1.4 Comparison with Us

| Item | CF-YOLO | Our project |
| --- | --- | --- |
| Base detector | YOLOv5s | YOLO26n |
| General dataset | MSCOCO | MIO-TCD |
| Snow dataset | RSOD | ACDC snow / Others |
| Snow images | 2,100 | 1,000~ / pending |
| Snow grading | Four difficulty levels | General and snow domain |
| General evaluation | Yes | Yes |
| Snow evaluation | Yes | Yes |
| Optimization method | Cross Fusion neck replacement | Weather-aware pruning |
| Accuracy | Better | Lower |
| Model size | Bigger | Smaller |
| Model speed | Slower | Faster |
| Reported hardware | Tesla V100 | Platogpu007 / PYNQ-Z2 |
| Reported efficiency | Params, GFLOPs, FPS | Params, size, latency, FPS, memory, CPU usage, (can be added) |

Table 1.10: project comparation

Table 1.10 shows: the main difference is that we have custom pruning, while they have custom fusion block.

1.5 What could be useful from CF-YOLO

Snow severity evaluated separately

Although we did not separate snow levels, we separated snow and non-snow.

General and snow performance

CF-YOLO is tested on both MSCOCO and RSOD. This is similar to the use of GEN and SNOW datasets.

Snow-specific data significantly improve performance

Training on RSOD improves CF-YOLO test AP50 by approximately 12–13. This confirms that snow-specific data provides information that may not be learned from a general dataset.

Lightweight architecture is not equal to model compression

CF-YOLO improves snow detection through a redesigned feature-fusion neck, but it has more parameters and lower FPS than YOLOv5s.

2. Competitor: Traffic YOLO Pruning (WCE, BDD100K) [2]

2.1 Main Contributions

Class aware channel pruning

The paper proposes a structured channel-pruning framework based on Weighted Copula Entropy, abbreviated as WCE. The method estimates the relationship between feature channels and object classes while increasing the importance of underrepresented classes.

Semantic importance

A channel can be retained if it contains useful information, even when its weight or activation magnitude is relatively small.

Removal of redundant channels

- relevant to the detection classes

- protective of rare classes

- less similar function redundant

Physical structured pruning

The selected channels are physically removed from the model.

2.2 BDD100K Dataset Usage

The study uses the BDD100K autonomous-driving dataset. According to the paper, BDD100K contains more than 100,000 driving videos and over 1,000 hours of driving footage. It covers different weather, lighting, urban/rural, daytime, and nighttime conditions.

| Class | Object instances | Dataset share |
| --- | --- | --- |
| Car | 1,021,857 | 55.49% |
| Traffic Sign | 343,777 | 18.67% |
| Traffic Light | 265,906 | 14.44% |
| Person | 129,262 | 7.02% |
| Truck | 42,963 | 2.33% |
| Bus | 16,505 | 0.90% |
| Bike | 10,229 | 0.56% |
| Rider | 6,461 | 0.35% |
| Motor | 4,296 | 0.23% |
| Train | 179 | 0.01% |
| Total | 1,841,435 | 100% |

Table 2.2: Class distribution

In Table 2.2, car alone accounts for more than half of all annotations, while the Train class contains only 179 examples. The paper gives the more precise Train proportion as approximately 0.0097%.

2.2.1 Relevance

The dataset distribution demonstrates why overall accuracy can hide weak performance on a smaller but important group.

For the WCE paper: High overall mAP (not imply) good rare-class detection

For our project: High GEN mAP (not imply) good SNOW detection

This analogy supports the decision to measure pruning effects separately instead of using only one overall validation score.

2.8 Class-Wise Results

The following table compares the unpruned baseline with the complete WCE-pruned model.

| Class | YOLOv8l Baseline -> WCE | Change | YOLOv5l Baseline -> WCE | Change |
| --- | --- | --- | --- | --- |
| Car | 80.25 -> 79.58 | −0.67 | 79.47 -> 78.85 | −0.62 |
| Traffic Sign | 70.18 -> 69.49 | −0.69 | 69.29 -> 67.95 | −1.34 |
| Traffic Light | 69.36 -> 68.67 | −0.69 | 68.50 -> 67.33 | −1.17 |
| Person | 66.02 -> 65.33 | −0.69 | 65.22 -> 63.87 | −1.35 |
| Truck | 59.87 -> 59.18 | −0.69 | 59.25 -> 58.22 | −1.03 |
| Bus | 57.63 -> 58.05 | +0.42 | 56.18 -> 56.31 | +0.13 |
| Bike | 52.89 -> 53.22 | +0.33 | 51.01 -> 51.11 | +0.10 |
| Rider | 49.45 -> 49.81 | +0.36 | 47.72 -> 47.77 | +0.05 |
| Motor | 48.26 -> 48.65 | +0.39 | 46.50 -> 46.55 | +0.05 |
| Train | 2.76 -> 5.12 | +2.36 | 0.00 -> 3.84 | +3.84 |

Table 2.7: Accuracy for each class

In Table 2.7, the results show two different patterns:

- Frequent classes such as Car, Traffic Sign, Traffic Light, Person, and Truck experience accuracy reductions.

- Several minority classes, including Bus, Bike, Rider, Motor, and Train, improve after pruning.

This suggests that the class-balanced criterion retains channels that are useful to rare classes but may damage common classes.

2.10 Ablation

The authors test four versions of their framework.

| Model | Variant | mAP@0.5 | FLOPs | Parameters |
| --- | --- | --- | --- | --- |
| YOLOv5l | Elastic Net only | 52.86 | 54.12 G | 22.51 M |
|  | WCE without class balancing | 53.51 | 53.78 G | 22.32 M |
|  | WCE + class balancing, without mRMR | 53.97 | 53.37 G | 22.16 M |
|  | Complete WCE framework | 54.18 | 53.23 G | 22.07 M |
| YOLOv8l | Elastic Net only | 54.53 | 89.73 G | 23.52 M |
|  | WCE without class balancing | 55.07 | 88.24 G | 23.09 M |
|  | WCE + class balancing, without mRMR | 55.59 | 87.09 G | 22.68 M |
|  | Complete WCE framework | 55.82 | 86.47 G | 22.86 M |

Table 2.8: frameworks

In Table 2.8, the results show the role of each component:

- Elastic Net pushes less useful channels toward small scaling

- WCE map channel effective to classes

- Class balancing improves adaptation to rare classes

- mRMR removes redundant features

The method combines the normalized Elastic Net score and WCE-mRMR score using a balance coefficient α.

The author’s test: α = 0.0, 0.1, 0.2, …, 1.0

Both YOLOv5l and YOLOv8l reach their highest mAP@0.5 at: α = 0.5

This means an equal balance between L1 L2 prune and proposed prune performs better than allowing either criterion to dominate the pruning decision.

This is relevant to GEN and SNOW sensitivity components should be balanced carefully.

2.12 Comparison with Us

| Item | WCE paper | Our project |
| --- | --- | --- |
| Detector | YOLOv5l and YOLOv8l | YOLO26n |
| General dataset | BDD100K | MIO-TCD |
| Snow dataset | None separately | ACDC Snow / (could be BDD100K) |
| Main imbalance | Frequent vs rare classes | GEN vs SNOW domains |
| Pruning unit | Channels (convolution) | Layers / channels / individual weights |
| Importance criterion | Elastic Net + WCE-mRMR | Measured GEN/SNOW accuracy drops |
| Main accuracy metric | mAP@0.5 (mAP50) | mAP50–95 |
| Parameter reduction | Approximately 48–52% | - |
| FLOP reduction | Approximately 48–51% | - |
| Per-weather evaluation | No | GEN and SNOW separately |
| Latency/FPS | Not reported | Measure on PYNQ-Z2 |
| Deployment hardware | RTX8000 | PYNQ-Z2 ARM |
| Main objective | Protect rare classes during pruning | Protect snow performance during pruning (while not ignoring up general) |

Table 2.9: project comparation

Table 2.9 shows our differences.

2.13 What could be useful from WCE

A small group should not be hidden by an aggregate metric

The paper demonstrates that a model can have good overall mAP while performing poorly on a rare but important traffic class.

For the current project, snow images represent a specialized condition that may similarly be hidden by overall GEN accuracy.

Importance should reflect task performance

WCE shows that weight magnitude alone may not identify whether a channel contains important semantic information.

Our framework takes this principle further by directly measuring the effect of removing a candidate on GEN and SNOW mAP50–95.

Balanced sensitivity can protect specialized features

WCE protects minority-class features through inverse-frequency weighting.

Our framework protects snow-sensitive components through separate SNOW evaluation and normalized accuracy drop.

3. Negative AD Reasoning

3.1 Removal of Domain-Sensitive or Harmful Features (CVPR) [3]

Wang et al. showed that some feature channels learned from the source domain are not universally useful. A channel may improve performance on the original source domain but reduce performance after a domain shift.

| Experiment | Direct test | Proposed method | Relevant result |
| --- | --- | --- | --- |
| Cityscapes -> Cityscapes-C, average mAP@50 | 4.5 | 11.4 | +6.9 mAP |
| Cityscapes-C, snow, round 1 | 0.6 | 2.6 | +2.0 mAP |
| Cityscapes-C, snow, round 5 | 0.6 | 4.6 | +4.0 mAP |
| Cityscapes-C, snow, round 10 | 0.6 | 4.8 | +4.2 mAP |
| Cityscapes -> ACDC, average mAP@50 | 22.0 | 24.2 | +2.2 mAP |
| ACDC snow, rounds 1 / 5 / 10 | 21.8 / 21.8 / 21.8 | 23.4 / 24.7 / 24.5 | Consistent snow improvement |

Table 3.1: suppress channels that changed strongly between domains & retaining invariant channels

For Table 3.1, this finding provides a particularly relevant explanation for a negative AD in our project.

For example, a pruned channel or pruning group may primarily encode general-weather textures, contrast patterns, backgrounds, or object appearances that are unreliable in snow. Removing that group can reduce misleading activations and improve mAP even though parameters have been removed. Such a group should not automatically be described as “unimportant”; it may instead be domain-sensitive (e.g. only useful on traffic, or snow, or pencils, etc.).

3.2 Pruning as Regularization and Noise Injection (GSTNNP) [4]

Bartoldson et al. explained another possible cause of negative AD: pruning can act as a form of regularization rather than only as model compression.

The authors describe this effect as being similar to noise injection regularization. Adding noise or temporarily disturbing a model can prevent it from relying too strongly on specific features. As a result, the model may reduce overfitting and perform better on unseen validation or test data.

The study found that an immediate accuracy reduction could sometimes lead to better final accuracy after continued training. This means that the temporary disturbance caused by pruning may encourage the model to find a simpler and more generalizable solution.

The results suggest that part of the accuracy improvement came from the disturbance-and-recovery process, rather than only from reducing the number of parameters.

4. Negative AD Handling

I suggest we retain negative values.

4.1 Support 1: Neuron Structured Pruning Polarization Regularizer (NSPPR) [5]

Their paper defines negative accuracy drop as a pruned model achieving higher accuracy than its unpruned baseline. Their reported pruned accuracy was measured after fine-tuning the pruned network.

| Dataset | Model | Baseline accuracy | Pruned accuracy | Accuracy drop | FLOPs reduction |
| --- | --- | --- | --- | --- | --- |
| CIFAR-10 | ResNet-56 | 93.80% | 93.83% | −0.03% | 47% |
| CIFAR-10 | VGG-16 | 93.88% | 93.92% | −0.04% | 54% |
| CIFAR-100 | VGG-16 | 73.83% | 74.25% | −0.42% | 43% |

Table 4.1: relevant negative-AD results

Table 4.1 shows part of their results.

4.2 Support 2: Learning Filter Pruning Criteria (LFPC) [6]

The authors state that a negative value indicates improved model accuracy. Therefore, the negative values were directly reported.

The LFPC method produced negative AD results for different ResNet models on CIFAR-10:

| Dataset | Model | Baseline accuracy | Pruned accuracy | Accuracy drop | FLOPs reduction |
| --- | --- | --- | --- | --- | --- |
| CIFAR-10 | ResNet-56 | 93.59% | 93.72% | −0.13% | 47.1% |
| CIFAR-10 | ResNet-110 | 93.68% | 93.79% | −0.11% | 60.3% |

Table 4.2: Relevant negative AD results from LFPC

Table 4.2 shows part of their results.

4.3 Support 3: Filter Pruning via Geometric Median (FPGM) [7]

FPGM removes filters that are considered redundant according to their distance from the geometric median of the filters. The experimental results show that removing redundant filters can reduce computation while maintaining or slightly improving model accuracy.

The paper reported several negative accuracy-drop results for ResNet models on CIFAR-10:

| Dataset | Model and pruning setting | Baseline accuracy | Pruned accuracy | Accuracy drop | FLOPs reduction |
| --- | --- | --- | --- | --- | --- |
| CIFAR-10 | ResNet-32, FPGM-mix 40% | 92.63% | 92.82% | −0.19% | 53.2% |
| CIFAR-10 | ResNet-110, FPGM-only 40% | 93.68% | 93.73% | −0.05% | 52.3% |
| CIFAR-10 | ResNet-110, FPGM-mix 40% | 93.68% | 93.85% | −0.17% | 52.3% |
| CIFAR-10 | ResNet-110, FPGM-only 40% with fine-tuning | 93.68% | 93.74% | −0.16% | 52.3% |

Table 4.3: Relevant negative AD results from FPGM

Table 4.3 shows part of their results.

4.4 Support 4: Visual-Saliency-Guided Channel Pruning [8]

Their study applied structured channel pruning to YOLOX-S and evaluated the resulting models on the KITTI traffic dataset.

| Dataset | Model | Parameters | FLOPs | mAP | Calculated AD |
| --- | --- | --- | --- | --- | --- |
| KITTI | Vanilla YOLOX-S | 8.94 M | 26.65 G | 87.3% | 0.0% |
| KITTI | L1-pruned YOLOX-S | 5.40 M | 18.71 G | 88.5% | −1.2% |
| KITTI | CHIP-pruned YOLOX-S | 5.35 M | 18.88 G | 88.9% | −1.6% |
| KITTI | Proposed pruned YOLOX-S | 5.35 M | 18.88 G | 89.1% | −1.8% |

Table 4.4: Relevant negative AD results for pruned YOLOX-S

In thble 4.4, The unpruned YOLOX-S achieved an mAP of 87.3%. At a pruning rate of 0.3, several pruning methods produced higher mAP values than the original model

4.5 Support 5: Streamlining YOLOv7 for Rapid and Accurate Detection [9]

The study developed a dual-dimensional pruning method that removes redundant structures in both the spatial and channel dimensions.

The final pruned YOLOv7 model achieved a slightly higher mAP than the original YOLOv7 baseline:

| Dataset/task | Model | Parameters | Inference time (Raspberry Pi 4B) | mAP | Calculated AD |
| --- | --- | --- | --- | --- | --- |
| Rapeseed variety detection | Original YOLOv7 | 36.50 M | 4.48 s/image | 96.68% | 0.0% |
| Rapeseed variety detection | Custom1 pruned YOLOv7 | 9.19 M | 1.18 s/image | 96.89% | −0.21% |

Table 4.5: Relevant negative AD result for pruned YOLOv7

Table 4.5 shows part of their results.

4.6 Conclusion

| Support | Architecture | Negative result | Resource reduction | Handling |
| --- | --- | --- | --- | --- |
| Polarization Pruning [5] | ResNet/VGG | −0.03% to −0.42% | Up to 54% FLOPs | Retained as improved accuracy |
| LFPC [6] | ResNet | −0.11% to −0.13% | Up to 60.3% FLOPs | Retained as improved accuracy |
| FPGM [7] | ResNet | −0.05% to −0.19% | Up to 53.2% FLOPs | Retained as performance improvement |
| Visual-saliency pruning [8] | YOLOX-S | −1.2% to −1.8% | 40.2% parameters and 29.2% FLOPs | Reported as mAP improvement |
| Dual-dimensional pruning [9] | YOLOv7 | −0.21% | 74.8% parameters and 73.7% inference time | Reported as surpassing baseline |

Table 4.6

I suggest we retain negative values, like how previous research did.

5. PYNQ-Z2 PS-Side Baseline Evaluation

5.2 Experimental Setup

All models were executed using the NCNN CPU inference framework on the PYNQ-Z2 PS side. The board used a 32-bit ARMv7 environment with two logical CPU cores.

The experiments used an input resolution of 640 × 640 pixels and two NCNN CPU threads.

| Setting | Value |
| --- | --- |
| Input resolution | 640 × 640 |
| Batch size | 1 |
| CPU threads | 2 |
| Warm-up inferences | 5 |
| Benchmark images | 50 |
| Repetitions | 3 |
| Total measured inferences | 150 |
| Confidence threshold | 0.25 |
| NMS IoU threshold | 0.70 |
| Maximum detections | 300 |

Table 5.1: benchmark configurations

About Table 5.1, the benchmark configuration was kept identical for all three models.

The reported compute latency includes image preprocessing, NCNN inference, output decoding, and non-maximum suppression.

Model loading and warm-up execution were excluded.

The FPS value was calculated from the mean compute latency.

5.3 Results

| Model | Training data | Classes | NCNN size (MiB) | Median latency (ms/image) | FPS | Peak memory (MiB) | CPU usage (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline Full-Data | Approximately 88,000 MIO-TCD images | 11 | 9.189 | 10,586.52 | 0.09469 | 132.96 | 95.27 |
| Baseline Reduced-Data | 7,700 MIO-TCD images | 11 | 9.189 | 10,592.06 | 0.09462 | 133.03 | 95.28 |
| Baseline Official | COCO | 80 | 9.311 | 11,042.63 | 0.09076 | 142.82 | 94.92 |

Table 5.2: PYNQ-Z2 PS NCNN performance

In Table 2.2, the full-data and reduced-data models required approximately 10.6 seconds to process one image (a lot!), corresponding to approximately 0.095 FPS. The official model required approximately 11.0 seconds per image and achieved approximately 0.091 FPS.

5.6 Bottleneck Analysis

For all three models, the main bottleneck was the NCNN network inference rather than image loading or post-processing.

| Stage | Mean time |
| --- | --- |
| Image reading | 22.83 ms |
| Preprocessing | 73.82 ms |
| NCNN inference | 10,485.54 ms |
| Post-processing | 1.86 ms |

Table 5.3: full-data model mean time

According to Table 2.3, NCNN inference contributed approximately 99.28% of the compute time. The normalized CPU usage was also approximately 95%, showing that the two ARM CPU cores were already heavily utilized.

Therefore, optimizing only image reading, preprocessing, or NMS would not provide a major speed improvement. The main reduction must come from decreasing the neural-network computation.

5.10 Conclusion

The PS-only NCNN evaluation showed that YOLO26n can be successfully executed on the PYNQ-Z2 ARM processor, but its current performance is not enough for real-time traffic detection.

The full-data and reduced-data models both achieved approximately 0.095 FPS because they use the same network architecture despite being trained with substantially different amounts of data. The official 80-class model was slightly slower and required more memory because of its larger detection output.

Overall, the results gave a software baseline and the need for actual model compression. Future experiments will compare these baseline results with pruned, quantized, etc. models using the same configuration.

6. Pruning: Layer ablation using parameter-free replacement

the original layer was replaced with a no-parameter adapter: E.g.

before pruning: Layer 8 -> Layer 9 -> Layer 10

After pruning: Layer 8 -> Adapter -> Layer 10

6.1 Baseline

The original GEN and SNOW models were evaluated using the server environment:

| Model | mAP50–95 | mAP50 |
| --- | --- | --- |
| GEN baseline | 0.6414 | 0.8300 |
| SNOW baseline | 0.1899 | 0.3241 |

Table 6.2: baseline mAP

For Table 6.2, these local results became the denominators for later Accuracy Drop calculations. (similar as Rafed’s result)

6.2 Remove one layer at a time

Layer 9, the SPPF layer, was the strongest individual candidate.

It gave approximately:

6.56% parameter reduction

Its raw accuracy changed as follows:

| Dataset | Baseline | After Layer 9 replacement |
| --- | --- | --- |
| GEN | 0.6414 | 0.5401 |
| SNOW | 0.1899 | 0.1558 |

Table 6.3: pruned layer 9

Table 6.3 shows that Layer 9 could be removed while preserving more accuracy than many other tested layers.

Other large layers, such as Layers 20 and 22, reduced more parameters but caused greater raw accuracy damage.

6.3 Test combinations of layers

The target was approximately 10.25% (5% of 10.25% var) parameter reduction.

The best raw combination was: C007 = Layers 9 and 19

This combination reduced approximately: 10.36% of the model parameters

Its raw accuracy was:

| Dataset | Baseline | Raw C007 |
| --- | --- | --- |
| GEN | 0.6414 | 0.0590 |
| SNOW | 0.1899 | 0.0761 |

Table 6.4: massive raw drop

Although the raw accuracy was low, C007 was still the strongest candidate among the combinations near the 10.25% target.

The candidate closest to 10.25% was C001:

- Layers 0, 2 and 10

- Reduction: approximately 10.24%

However, its GEN and SNOW mAP50–95 values were both zero. Therefore, it was rejected.

I can conclude that the raw accuracy drop will be very high when trying to prune more than one whole layer at the same time.

A single Layer 9 replacement was kept as a single layer control model.

| Candidate | Removed layers | Purpose |
| --- | --- | --- |
| C007 | 9 and 19 | Param drop near target |
| L9 | 9 | Single-layer control |

Table 6.5: recovery sets

Table 6.5 shows the two candidates selected for recovery.

6.4 Fine-tune and recover

Removing layers damaged the pretrained feature processing. Fine-tuning could teach the remaining model to work without those layers. This process is called recovery training. The main settings of fine-tuning were:

| Setting | Value |
| --- | --- |
| Epochs | 20 |
| Image size | 640 |
| Batch size | 16 |
| Optimizer | AdamW |
| Warm-up | 1 epoch |
| Pretrained reload | Disabled |

Table 6.6: settings

In table 6.6, pretrained=False did not mean that training started from a random model. The replacement checkpoint had already been loaded. It meant that Ultralytics should not load another pretrained model over the modified architecture.

6.5 Results Comparing (with Rafed)

Method A: Whole-Layer Replacement

This method removes the entire YOLO layer and replaces it with a simple no-parameter adapter.

Method B: Dependency-Group Channel Pruning

This method removes selected channels, not removing an entire layer.

A fair comparison uses:

- Approximately the same parameter reduction

- The same GEN and SNOW datasets spilt

- The same image size and evaluation settings

- The same fine-tuning budget

- The final recovered models

Both C007 and T6 used approximately 10.25% parameter reduction and 20 epochs of recovery training.

Therefore, they provide the closest current comparison.

Both methods lost substantial accuracy immediately after pruning.

| Raw pruned model | GEN retention | SNOW retention |
| --- | --- | --- |
| Whole-layer L9 | 84.20% | 82.06% |
| Whole-layer C007 | 9.21% | 40.05% |
| Dependency-group T5 | 30.81% | 44.80% |

Table 6.7: raw pruning result

Table 6.7 shows the dependency-group model suffered less immediate GEN damage.

This is understandable because channel pruning preserves most of each layer, while whole-layer replacement completely removes the feature-processing operation of the selected layer.

However, both experiments also show that fine-tuning is essential. The final result should therefore be judged using C007 recovered and T6 recovered, not their raw pruned versions

The two experiments used slightly different measured baseline values. For this reason, accuracy retention % is more suitable than comparing only the raw mAP values (how much of the original model’s accuracy remains after pruning and fine-tuning).

| Method | Parameters reduced | GEN mAP50–95 | GEN retention | SNOW mAP50–95 | SNOW retention |
| --- | --- | --- | --- | --- | --- |
| Whole-layer replacement, L9 | 6.56% | 0.6347 | 98.95% | 0.1908 | 100.47% |
| Whole-layer replacement, C007 | 10.36% | 0.6184 | 96.42% | 0.1807 | 95.14% |
| Dependency-group pruning, T6 | 10.25% | 0.6371 | 98.45% | 0.1818 | 94.72% |

Table 6.8: param & retention compare

In Table 6.8, C007 did slightly better on SNOW, while T6 did better on GEN.

The average retention across GEN and SNOW was:

- C007: approximately 95.78%

- T6: approximately 96.59%

Their results are close.

| Aspect | Whole-layer replacement | Dependency-group channel pruning |
| --- | --- | --- |
| Pruning unit | Complete top-level layer | Selected connected channels |
| Method complexity | Simpler pruning decision | More complex dependency management |
| Control of reduction | Difficult because layers have fixed sizes | Easier to adjust gradually |
| Immediate accuracy damage | Can be very large | Usually more controlled |
| Experience parameters reduced | More | Very close to C007 |
| Experience GEN result | Lower | Better |
| Experience SNOW result | Better | Very close to C007 |
| Model structure | Using a custom no-parameter adapter | Physically reduces channels in existing structures |

Table 6.9: Advantages and disadvantages

In table 6.9, no matter which method we use, the other method can be an additional experiment used to show what happens when a different design choice is used (ablation experiment).

Rafed’s report also describes whole-module experiments as useful diagnostic or competitor evidence, while presenting physical dependency-group pruning as the main deployable compression method.

7. Formulars suggestions

Since three of our thoughts on the formula are quite different, I think we have two options.

7.1 Option 1: compare the results of different formulas

Initially, we got:

NADGEN,i=mAPGEN,baseline-mAPGEN,pruned,imAPGEN,baseline

NADSNOW,i=mAPSNOW,baseline-mAPSNOW,pruned,imAPSNOW,baseline

NADGEN,i≤0.05  and  NADSNOW,i≤0.05

Parameters%i=ParametersiTotal Model Parameters×100

Formula A:

Si=αNADGEN,i+(1-α)NADSNOW,i

Pi=Parameters%iSi

Formula B:

Snew,i=1NADGEN,i-NADSNOW,i

Pi=Parameters%iSi

Formula C:

Si=max⁡NADGEN,i NADGEN,i+NADSNOW,i2

Bi=0.5×RParameters,i+0.5×RGFLOPs,i

Pi=Bi1+Siτ2 , τ=0.05

For formula C, the completed equation can be explained as:

- rewards useful resource reduction while strongly penalizing groups that cause accuracy loss

The result of comparation can be our contribution, states that we tested the results on two equations.

The one with better outcomes will then be used, and the final model will depend on it.

7.2 Option 2: Ebrahim determines the final formula

It may take a long while for the three of us to reach a consensus. Letting one person make the final decision is the most efficient way.

8. Summary

Competitor: Snow Aware (CF YOLO)

- CF-YOLO improves snowy-condition detection by replacing the YOLOv5s neck with Cross Fusion blocks that combine features from multiple backbone levels.

- Snow-specific training produces large accuracy improvements, which supports evaluating and training the model separately on the GEN and SNOW datasets.

Competitor: Traffic YOLO Pruning (BDD100K)

- The WCE method performs structured channel pruning while protecting features that are important to rare traffic-object classes.

- WCE also demonstrated that a good overall mAP does not guarantee good performance for smaller but important data groups. Supports GEN–SNOW framework because general traffic data should not dominate the pruning decision and hide damage to snow-related features.

Negative AD Reasoning & Handling

- Negative AD may occur when pruning removes redundant, overfitted, domain-sensitive, or harmful features from the model.

- Pruning can also act as regularization by temporarily disturbing the model and encouraging it to learn a simpler and more generalizable solution during fine-tuning.

- Multiple structured-pruning studies, including YOLOX-S and YOLOv7 experiments, retain negative AD values and interpret them as genuine accuracy improvements.

Whole-Layer Replacement vs Dependency-Group Channel Pruning

- Dependency-Group Channel Pruning wins on raw accuracy.

- Their results are very close after fine-tuning.

PYNQ-Z2 PS-Side Baseline Evaluation

- The PYNQ-Z2 PS-side baseline requires approximately 10–11 seconds per image and achieves only about 0.09 FPS, which is not enough for real-time traffic detection.

- The full-data and reduced-data models have almost identical latency because changing the amount of training data does not change the model architecture or computational cost.

- NCNN inference contributes more than 99% of the total processing time, so the main optimization must reduce neural-network computation (not preprocessing or post-processing).

Pruning

- The results of Dependency-Group Channel Pruning and Whole Layer Raplacement are close.

Formular

- Competing formulas could be compared experimentally, or make the decision by one person immediately.

References

[1] Ding, Q., Li, P., Yan, X., Shi, D., Liang, L., Wang, W., Xie, H., Li, J., & Wei, M. (2023). CF-YOLO: Cross fusion YOLO for object detection in adverse weather with a high-quality real snow dataset. IEEE Transactions on Intelligent Transportation Systems, 24(10), 10749–10759. https://doi.org/10.1109/TITS.2023.3285035

[2] Zhou, Y., Ma, J., & Dong, H. (2026). Weighted copula entropy for structural pruning in long-tailed autonomous driving object detection. Entropy, 28(3), Article 336. https://doi.org/10.3390/e28030336

[3] Wang, K., Fu, X., Lu, X., Ge, C., Cao, C., Zhai, W., & Zha, Z.-J. (2025). Efficient test-time adaptive object detection via sensitivity-guided pruning. In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition (pp. 10577–10586). https://doi.org/10.1109/CVPR52734.2025.00989

[4] Bartoldson, B. R., Morcos, A. S., Barbu, A., & Erlebacher, G. (2020). The generalization-stability tradeoff in neural network pruning. Advances in Neural Information Processing Systems, 33. https://proceedings.neurips.cc/paper/2020/hash/ef2ee09ea9551de88bc11fd7eeea93b0-Abstract.html

[5] Zhuang, T., Zhang, Z., Huang, Y., Zeng, X., Shuang, K., & Li, X. (2020). Neuron-level structured pruning using polarization regularizer. Advances in Neural Information Processing Systems, 33. https://proceedings.neurips.cc/paper/2020/hash/703957b6dd9e3a7980e040bee50ded65-Abstract.html

[6] He, Y., Ding, Y., Liu, P., Zhu, L., Zhang, H., & Yang, Y. (2020). Learning filter pruning criteria for deep convolutional neural networks acceleration. In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition (pp. 2009–2018). https://openaccess.thecvf.com/content_CVPR_2020/html/He_Learning_Filter_Pruning_Criteria_for_Deep_Convolutional_Neural_Networks_Acceleration_CVPR_2020_paper.html

[7] He, Y., Liu, P., Wang, Z., Hu, Z., & Yang, Y. (2019). Filter pruning via geometric median for deep convolutional neural networks acceleration. In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition (pp. 4340–4349). https://doi.org/10.1109/CVPR.2019.00447

[8] Choi, J. I., & Tian, Q. (2023). Visual-saliency-guided channel pruning for deep visual detectors in autonomous driving. In Proceedings of the 2023 IEEE Intelligent Vehicles Symposium (IV) (pp. 1–6). https://doi.org/10.1109/IV55152.2023.10186819

[9] Gu, S., Meng, W., & Sun, G. (2024). Streamlining YOLOv7 for rapid and accurate detection of rapeseed varieties on embedded device. Sensors, 24(17), Article 5585. https://doi.org/10.3390/s24175585