# measurement for NCNN models on the PYNQ-Z2 PS

Jupyter Notebook
      ↓
measure_model.sh
      ↓
run_model.sh
      ↓
yolo26_ncnn_runner.cpp
      ↓
NCNN model runs on PYNQ-Z2
      ↓
CSV results

# 1. Jupyter notebook: selects the model and test settings

MODEL_NAME: model to test.

MODE: benchmark, validate, or all.

IMAGE_LIMIT = 1: quick one-image smoke test; 0: full test.

IMGSZ: input resolution, such as 640, 480, or 320.

# 2. measure_model.sh: checks the model folder contains

model.ncnn.param
model.ncnn.bin
metadata.yaml

# 3. run_model.sh: records the NCNN size as

NCNN deployment size
= model.ncnn.param size
+ model.ncnn.bin size

The size is saved in model_size.csv. The script also saves run_config.txt with:

model name and test mode;

input resolution and image limit;

CPU architecture and logical core count;

number of CPU threads.

Deployment file size is not the same as the model parameter count.

# 4. Same Benchmark Settings for Every Model

benchmark_images.txt
2 CPU threads
5 warm-up runs
3 repetitions
Confidence threshold = 0.25
IoU threshold = 0.70
Maximum detections = 300

50 benchmark images and 3 repetitions = 150 measured inferences.

# 5. yolo26_ncnn_runner.cpp: measures for each image

Image reading
    ↓
Preprocessing
    ↓
NCNN inference
    ↓
Post-processing

## A. Image I/O

OpenCV reads the image from storage into memory. This time is recorded as io_ms.

## B. Preprocessing

The image is resized while keeping its aspect ratio, padded to a square, converted from BGR to RGB, and normalized from 0-255 to 0-1. This time is recorded as preprocess_ms.

## C. NCNN Inference

The prepared image is sent into the NCNN YOLO model through input blob in0, and the output is read from out0. This is the neural-network computation and is recorded as inference_ms.

## D. Post-processing

The runner decodes bounding boxes and classes, removes low-confidence detections, restores coordinates to the original image, applies class-aware NMS to remove duplicate boxes, and limits the final number of detections. This time is recorded as postprocess_ms.

# 6. Latency Calculated

Compute latency
= Preprocessing
+ NCNN inference
+ Post-processing

Image reading is measured separately and is not included in compute latency.

| Metric | Includes |
| --- | --- |
| Compute latency | Preprocess + inference + post-process |
| Total latency | Image I/O + compute latency |

# 7. FPS Calculated

Compute FPS = 1000 / Mean compute latency (ms)

The runner also calculates total FPS using clock time, including image reading.

# 8. Median and P95 Latency

The runner stores the latency of every measured inference and calculates percentiles from those values.

| Metric | Simple meaning |
| --- | --- |
| Median (50th percentile) | Half the runs are faster and half are slower. |
| P95 (95th percentile) | 95% of runs finish within this time; stability. |

# 9. Memory Measured

The runner uses Linux getrusage() and reads ru_maxrss: highest memory used by the process.

Peak RSS memory (MiB) = ru_maxrss / 1024

Peak RSS shows the largest amount of physical RAM used during the test.

# 10. CPU Usage Measured

The runner records the process CPU time before and after the benchmark.

Raw process CPU % = CPU time / elapsed time × 100
Normalized board CPU % = Raw process CPU % / logical CPU cores

PYNQ-Z2 has two logical CPU cores, raw process CPU usage can approach 200%.

# 11. Saved

2_results/<model_name>/
├── performance.csv
├── model_size.csv
├── run_config.txt
├── metadata.yaml
└── model_hashes.txt

performance.csv contains the main benchmark results:

mean I/O, preprocessing, inference, and post-processing time;

mean, median, and P95 compute latency;

compute FPS and total FPS;

peak memory and CPU usage;

output tensor dimensions.

# Whole Method

NCNN model (.param + .bin + metadata)
              ↓
Check files and record hashes
              ↓
Record model size + test settings
              ↓
Load model into NCNN on PYNQ-Z2 PS
              ↓
5 warm-up runs (not measured)
              ↓
50 fixed benchmark images × 3 repeats
        = 150 measured runs
              ↓
Read image → Preprocess → NCNN inference → Post-process
              ↓
Calculate latency, FPS, memory, and CPU usage
              ↓
performance.csv

PC-PYNQ Live Video Inference

# Core Files

PC: stream_video_frame_skip.py - video input, resizing, TCP transmission, frame skipping, box drawing, and display.

PYNQ: yolo26_ncnn_server.cpp - TCP server, image decoding, preprocessing, NCNN inference, NMS, and result return.

Support files:

run_live_server.sh - selects the model and starts the PYNQ server.

CMakeLists.txt - compiles the C++ server and runner.

model.ncnn.param + model.ncnn.bin - deployed NCNN model structure and weights.

metadata.yaml - model metadata, including class names.

# Quick Step-to-File Reference

| Step | Action | Main file | Main code / function |
| --- | --- | --- | --- |
| 1 | PC reads video | stream_video_frame_skip.py | cv2.VideoCapture(), capture.read() |
| 2 | PC resizes frame | stream_video_frame_skip.py | cv2.resize(), cv2.imencode() |
| 3 | PC sends frame to PYNQ | stream_video_frame_skip.py | socket.create_connection(), sock.sendall() |
| 4 | PYNQ runs YOLO detection | yolo26_ncnn_server.cpp | cv::imdecode(), preprocess_image(), run_inference() |
| 5 | PYNQ returns detections | server.cpp + Python client | send_exact(), receive_result() |
| 6 | PC draws/displays boxes | stream_video_frame_skip.py | cv2.rectangle(), cv2.putText(), cv2.imshow() |
| Extra | Frame skipping | stream_video_frame_skip.py | capture.grab() |
| Extra | Select model | run_live_server.sh | 2_baseline_full / 4_layer_replacement_320_FP16 |

# 1. PC Reads the Video

File: stream_video_frame_skip.py

capture = cv2.VideoCapture(args.video)
...
ok, frame = capture.read()

# 2. PC Resizes the Frame

File: stream_video_frame_skip.py

send_frame = cv2.resize(...)
cv2.imencode(".jpg", send_frame, ...)

# 3. PC Sends the Frame to PYNQ

PC file: stream_video_frame_skip.py

socket.create_connection((args.host, args.port))
...
sock.sendall(request_header)
sock.sendall(payload)

The TCP message contains the frame ID, JPEG size, and JPEG image data.

## PYNQ receives the frame

PYNQ file: yolo26_ncnn_server.cpp

socket(...)  ->  bind(...)  ->  listen(...)  ->  accept(...)
receive_exact(...)
cv::imdecode(...)

The PYNQ listens on port 5000, accepts the PC connection, receives the JPEG bytes, and converts them back into an image.

# 4. PYNQ Runs YOLO Detection

File: yolo26_ncnn_server.cpp

JPEG image -> decode -> preprocess -> YOLO/NCNN inference -> NMS -> detections

## 4.1 Model loading

network.load_param(...)
network.load_model(...)

The model is loaded once when the server starts, then the same NCNN network is reused for every processed frame.

# 5. Model Files Actually Used

/home/xilinx/yolo26_ps/1_models/4_layer_replacement_320_FP16/
  model.ncnn.param
  model.ncnn.bin
  metadata.yaml

| File | Purpose |
| --- | --- |
| model.ncnn.param | NCNN model structure / layer configuration |
| model.ncnn.bin | Trained model weights |
| metadata.yaml | Model metadata, including class names |

# 6. Which File Selects the Model?

File: run_live_server.sh

The script selects the model directory and input size, then starts yolo26_ncnn_server.

bash 1_scripts/run_live_server.sh 4_layer_replacement_320_FP16

# 7. PYNQ Preprocessing

File: yolo26_ncnn_server.cpp

received image
  -> resize
  -> letterbox padding (320x180 -> 320x320)
  -> BGR to RGB
  -> normalize 0-255 to 0-1
  -> NCNN input

# 8. Actual NCNN Inference

File: yolo26_ncnn_server.cpp: run_inference()

extractor.input("in0", input);
extractor.extract("out0", output);

The input call provides the preprocessed image to the network.

The extract call runs the NCNN model and obtains the YOLO output.

# 9. PYNQ Postprocessing

File: yolo26_ncnn_server.cpp: class_aware_nms()

YOLO numeric output
  -> decode predictions
  -> confidence filtering
  -> NMS
  -> final detections

Each final detection contains a class ID, confidence score, and bounding-box coordinates (x1, y1, x2, y2).

# 10. PYNQ Sends Results Back

File: yolo26_ncnn_server.cpp

RESULT <frame_id> <timing...> <count>
DET <class_id> <confidence> <x1> <y1> <x2> <y2>
...
END

PYNQ returns only detection and timing data; does not send the processed image back to the PC.

# 11. PC Receives Detection Results

File: stream_video_frame_skip.py: receive_result()

timing, detections = receive_result(sock, frame_id)

PC parses RESULT and DET records and checks that the returned frame ID matches the frame it sent.

# 12. PC Maps Boxes Back to the Original Video

File: stream_video_frame_skip.py

x_scale = original_width / send_width
y_scale = original_height / send_height

PYNQ receives a smaller frame, PC scales the returned coordinates back to the original video size.

# 13. PC Draws Bounding Boxes

File: stream_video_frame_skip.py

cv2.rectangle(...)
cv2.putText(...)

# 14. PC Displays the Result

File: stream_video_frame_skip.py

display_frame = cv2.resize(...)
cv2.imshow("PYNQ-Z2 YOLO26 NCNN", display_frame)

resized: 3840x2160 to 1280x720.

# 15. Frame Skipping

File: stream_video_frame_skip.py

capture.grab()

# File-Level Architecture

PC: stream_video_frame_skip.py
  Read video / frame skip / resize / JPEG encode
                  |
              TCP / Ethernet
                  v
PYNQ: yolo26_ncnn_server.cpp
  Decode JPEG -> preprocess -> NCNN YOLO model
                               |
                 model.ncnn.param + model.ncnn.bin
                               |
                         Decode + NMS
                               |
                     Detection results
                               |
              TCP / Ethernet back to PC
                               v
PC: receive_result() -> scale coordinates -> draw -> display

# Supporting File Notes

## run_live_server.sh

Chooses the model, input size, thread count, port, confidence threshold, IoU threshold, and maximum detections, then launches yolo26_ncnn_server.

## CMakeLists.txt

Used only to compile the C++ programs and link NCNN, OpenCV, OpenMP, and Threads. It is not part of per-frame inference.

yolo26_ncnn_server.cpp + NCNN/OpenCV/OpenMP/Threads -> CMake/make -> yolo26_ncnn_server