FRIDAY PROGRESS REPORT

Unified Conv Engine and INT8 DSP Reduction

Reporting date: Friday, July 10, 2026

| Project | Lightweight YOLO object detection on PYNQ-Z2 FPGA |
| --- | --- |
| Current focus | Runtime-configurable unified Conv1x1/Conv3x3 engine with INT8 DSP reduction |
| Target board | PYNQ-Z2 / xc7z020clg400-1; KV260 remains a future higher-headroom option |
| Evidence boundary | CSim, HLS synthesis, Vivado implementation, and bitstream generation are reported separately. No PYNQ board execution has been completed yet. |

# Executive Summary

Main result The unified convolution direction is now technically meaningful: one runtime-configurable engine handles Conv1x1 and Conv3x3, and the INT8 LUT-multiplier variant has reached Vivado implementation and bitstream generation while using only 42 DSP48E after implementation.

The fixed-point unified engine passed CSim on real YOLO26n layer22 Conv1x1 and Conv3x3 golden tensors and passed HLS synthesis/IP packaging.

The INT8 unified engine passed CSim on the same two real layer cases using testbench-derived symmetric INT8 quantization.

The INT8 LUT-multiplier variant was implemented in Vivado 2018.2; final impl_2 with Performance_Explore met 100 MHz timing with WNS 0.151 ns and generated a bitstream.

Board-level validation remains pending because PYNQ/Kria board access is not yet available.

# 1. Unified Conv Engine Work Completed

Target. Move away from separate Conv1x1 and Conv3x3 IP blocks and toward one runtime-configurable accelerator that can be reused by software scheduling. The kernel type, stride, padding, and dimensions are selected through AXI-Lite registers, so the FPGA does not need a new bitstream when switching between Conv1x1 and Conv3x3.

Why this matters. The previous dual-IP direction was too DSP-heavy for a PYNQ-Z2 system-level design. A shared physical datapath is a cleaner architecture for PYNQ-Z2 because it reduces concurrent hardware duplication and leaves more resource budget for AXI interconnect, control, buffering, and future blocks.

| Engine | Runtime support | Validation completed | Evidence boundary |
| --- | --- | --- | --- |
| Fixed-point unified v1 | Conv1x1, Conv3x3, stride 1/2 where applicable, padding, activation enable | Synthetic CSim, 2 real golden tensor CSim cases, HLS synthesis, IP packaging | No Vivado implementation yet |
| INT8 unified default-DSP | Same runtime engine with INT8 activations/weights and INT32 bias/requantization | 2 real golden tensor CSim cases, HLS synthesis, IP packaging | No Vivado implementation yet |
| INT8 unified LUT-multiplier | Same INT8 runtime interface, compiled with LUT-multiplier variant | 2 real golden tensor CSim cases, HLS synthesis, IP packaging, Vivado implementation, bitstream | No board execution yet |

# 2. Resource Comparison

Comparison rule. The first table compares HLS synthesis estimates for the three unified designs. The Vivado row is shown separately because post-implementation utilization is a different evidence level from HLS synthesis.

| Design | Stage | Est. clock | BRAM_18K | DSP48E | FF / LUT | Status |
| --- | --- | --- | --- | --- | --- | --- |
| Fixed-point unified | HLS | 8.750 ns | 146 (52%) | 81 (36%) | 8,754 / 13,021 | CSim + HLS + IP |
| INT8 unified | HLS | 8.750 ns | 130 (46%) | 71 (32%) | 17,899 / 19,769 | CSim + HLS + IP |
| INT8 + LUT multiplier | HLS | 8.750 ns | 130 (46%) | 39 (17%) | 18,412 / 23,001 | CSim + HLS + IP |

| Implemented design | Run | Timing | BRAM tiles | DSP48E | FF / LUT | Power |
| --- | --- | --- | --- | --- | --- | --- |
| INT8 + LUT multiplier | Vivado impl_2, Performance_Explore | WNS 0.151 ns, TNS 0.000 ns | 65 / 140 (46.43%) | 42 / 220 (19.09%) | 19,462 / 15,817 | 1.968 W |

Interpretation. At HLS level, the LUT-multiplier INT8 variant reduces DSP use from 81 to 39 compared with the fixed-point unified engine, a 51.9% reduction. After Vivado implementation, the LUT-multiplier variant uses 42 DSP48E and still meets 100 MHz timing. The tradeoff is higher LUT/FF use and unchanged BRAM pressure, which is expected because local buffering and maximum compile-time bounds still dominate BRAM.

# 3. INT8 Work and Validation Status

| Golden tensor case | Kernel | Shape | CSim | Max abs error vs FP32 | Mean abs error |
| --- | --- | --- | --- | --- | --- |
| layer22 C3k2 internal Conv1x1 | 1x1 | 1x384x20x20 -> 1x256x20x20 | Passed | 0.0598524 | 0.0101449 |
| layer22 C3k2 internal Conv3x3 | 3x3 | 1x128x20x20 -> 1x64x20x20 | Passed | 0.0543421 | 0.0104062 |

Important boundary. These INT8 tests use temporary symmetric scales derived inside the testbench from FP32 golden tensors. This verifies the hardware arithmetic contract, tensor layout, AXI interface direction, bias handling, and requantization path. It is not final QAT model accuracy evidence. Final accuracy evidence requires QAT-exported INT8 tensors and quantization parameters from the software track.

# 4. DSP-Reduction Methods Used

| Method | How it reduces DSP pressure | Current evidence |
| --- | --- | --- |
| Unified physical datapath | One accelerator handles Conv1x1 and Conv3x3 at runtime instead of keeping separate physical Conv1x1 and Conv3x3 datapaths active in the overlay. | Fixed-point unified HLS uses 81 DSP; previous separate planning point used 214 DSP after implementation. |
| INT8 arithmetic path | Moves activations and weights to 8-bit values with INT32 accumulation/requantization, enabling smaller multiply logic and future QAT compatibility. | INT8 default HLS uses 71 DSP versus 81 DSP for fixed-point unified. |
| LUT-multiplier variant | Compiles the INT8 multiplier path to reduce DSP inference and shift arithmetic cost into LUT fabric. | HLS DSP falls to 39; Vivado post-implementation uses 42 DSP and meets timing. |
| Runtime reuse instead of per-layer IP | Keeps one reusable engine and changes layer parameters through AXI-Lite registers, avoiding a one-IP-per-layer architecture. | Engine supports runtime Conv1x1/Conv3x3 selection and the layer22 1x1/3x3 golden cases. |
| PYNQ-aware resource target | Optimizes for a running full-system path, not only isolated kernel latency, so DSP headroom is preserved for control/interconnect and future blocks. | INT8 LUT-multiplier implementation uses 19.09% DSP on PYNQ-Z2. |

# 5. Required QAT INT8 Handoff From Software Track

Target for next week. The hardware INT8 interface is ready for final-style values. The next blocker is receiving a QAT INT8 model package whose tensors and quantization metadata can be plugged into the HLS testbench and then the PYNQ driver.

| Needed item | Preferred format | Why hardware needs it |
| --- | --- | --- |
| QAT model checkpoint | .pt or .pth plus model config | Confirms the exact trained/pruned architecture, layer names, channel counts, and activation policy. |
| Exported INT8 ONNX | .onnx with Q/DQ nodes or equivalent quant metadata | Provides a portable model graph and lets hardware mapping stay synchronized with the software model. |
| Per-layer INT8 weights | .npz or raw binary, OIHW layout, signed int8 | Direct input for hardware weight memory and CSim/PYNQ tests. |
| Bias values | Signed int32 per output channel, already in accumulator units | Avoids ambiguity in fixed-point scaling and matches the hardware accumulator path. |
| Activation tensors | Representative input/output tensors in NCHW layout | Allows golden tensor validation for selected Conv1x1 and Conv3x3 layers. |
| Quantization parameters | Input/output scales, zero-points, per-channel weight scales, requant multipliers and shifts | Needed to reproduce QAT inference numerically in the FPGA engine. |
| Preprocessing details | Image size, letterbox/pad policy, normalization, channel order | Prevents mismatch between software tensors and hardware input buffers. |
| Accuracy report | mAP50, mAP50-95, validation set/version, calibration/QAT recipe | Confirms that DSP reduction does not destroy model quality. |
| Layer mapping CSV | Layer name, type, shape, kernel, stride, padding, activation | Lets hardware scripts regenerate shapes, schedules, and golden tests without manual mapping errors. |

# 6. Essential SW-Side Techniques

| SW-side technique | Hardware resource helped | Reason this is required from SW side |
| --- | --- | --- |
| QAT INT8 training | DSP, DDR bandwidth, model accuracy under INT8 | Post-training fake INT8 tests are not enough; hardware needs a model trained to tolerate 8-bit activations/weights. |
| Input size reduction, e.g. 640 -> 320 if accuracy allows | Latency, DDR traffic, activation memory | Spatial dimensions drive the number of output pixels and therefore total MACs and buffer traffic. |
| Structured channel pruning / width multiplier | BRAM, DSP, LUT/FF, latency | Channel counts drive local weight storage and multiply count; structured pruning can reduce compile-time hardware bounds. |
| Channel alignment to 16/32 where possible | Lane utilization and latency | Avoids wasting parallel output lanes and reduces tail-channel overhead. |
| BatchNorm folded into Conv | Runtime operations and memory traffic | Hardware should receive fused weights/bias so it does not implement separate BN layers. |
| Simpler activation policy or activation clipping | LUT/logic, quantization accuracy | SiLU is more expensive than ReLU/clamp-style activations; clipping also improves INT8 range control. |
| Reduce deep-layer channel caps | BRAM, weight storage, latency | High-channel deeper layers dominate local weight buffers and MAC count. |
| Reduce C3K2/CSP repeat depth if accuracy permits | Latency, weights, DDR transfers | Fewer repeated blocks means fewer accelerator calls and less intermediate feature-map movement. |
| Depthwise-separable replacement for selected 3x3 convs | MAC count and weights | Dense 3x3 layers are expensive; depthwise-separable convs can reduce compute if the model remains accurate and hardware support is added. |

# 7. Targets for Next Week

Coordinate with the SW intern and supervisor to define the exact QAT INT8 handoff package listed above.

Prepare a plug-in script/testbench path that can load QAT INT8 weights, biases, activation tensors, scales, zero-points, multipliers, and shifts without changing the HLS kernel source.

Optionally run Vivado implementation for the INT8 default-DSP unified variant so the DSP-vs-LUT tradeoff can be compared at the same evidence level.

Build fixed-shape latency wrappers for one Conv1x1 and one Conv3x3 layer if concrete per-layer INT8 cycle estimates are needed for reporting.

Start preparing the PYNQ overlay folder layout and driver checklist for the INT8 LUT-multiplier bitstream and HWH.

Continue trying to obtain board access through the NUST lab so PYNQ overlay validation can begin locally.

# 8. Board Access and PYNQ Overlay Plan

Current status. The unified INT8 LUT-multiplier overlay files exist, but no board execution has been performed yet. The next board-level step is to load the generated bitstream and HWH, allocate PYNQ buffers, pass real golden tensors through the accelerator, and compare the result against expected outputs.

NUST lab plan. I will continue trying to get access to a PYNQ-Z2 board through a NUST lab. Once board access is available, the immediate target is correctness first: successful overlay load, register programming, buffer transfer, accelerator completion, output comparison, and then measured board latency.

# Evidence Files

| Artifact | Path |
| --- | --- |
| Fixed-point unified HLS evidence | YOLO26N/mio_baseline_hw/runs/hls/conv_unified_runtime_v1/README.md |
| INT8 unified HLS evidence | YOLO26N/mio_baseline_hw/runs/hls/conv_unified_int8_runtime_v1/README.md |
| INT8 LUT Vivado summary | YOLO26N/mio_baseline_hw/vivado_results/unified_int8_lutmul/README.md |
| INT8 LUT post-implementation CSV | YOLO26N/mio_baseline_hw/vivado_results/unified_int8_lutmul/post_implementation_summary.csv |
| Bitstream | YOLO26N/mio_baseline_hw/vivado_results/unified_int8_lutmul/unified_int8_lutmul.runs/impl_2/design_1_wrapper.bit |
| HWH | YOLO26N/mio_baseline_hw/vivado_results/unified_int8_lutmul/unified_int8_lutmul.srcs/sources_1/bd/design_1/hw_handoff/design_1.hwh |